AT03 GPRS Data Protocol

This is the AT03 GPRS data format communication protocol, this protocol is defined to make available a datagram mode of packet communication, It provides a procedure for application programs to send messages to other device.

Command Format Specification

Note: All multi-byte data is based on high-byte first, low-byte after for organization in this protocol.

| Code | Explanation |
| --- | --- |
| $$ | 2Bytes, indicates header of command from tracker unit to call centre, in ASCII code (hex is 0x24). |
| Len | 2Bytes, indicates length of all command, including header and end (the array is first high to low). |
| IMEI | 15Bytes, at most 20 bytes. |
| Alarm type | 2Bytes, the GPRS data trigger type. |
| DATA | GPRMC string |
|  | PDOP |
|  | HDOP |
|  | VDOP |
|  | Status (12bytes) |
|  | RTC (14bytes) |
|  | Voltage(8bytes) |
| ADC | 8bytes, the ADC value. |
| ADC percent | 6bytes,the percent of the ADC |
| LACCI | Location information elements (AGPS) |
| Temperature | Temperature information |
| Milemeter | Mileage data |
| Serial ID | 4bytes, sign every GPRS data, the range is [0001-9999], then circle it again from 0001 to 9999. |
| Checksum | 2Bytes, means CRC check of all the data ahead, CRC-16 modbus (Polynomial = 0xA001，initialize data is 0xffff) checksum, not including its own byte and end characters. For example: (hex format） 24 24 00 11 13 61 23 45 67 8f ff 50 00 80 43 0d 0a 0x8043 = CRC-16 modbus (24 24 00 11 13 61 23 45 67 8f ff 50 00). |
| \r\n | 2Bytes, end char (hex format is 0x0d,0x0a). |

Format：$$(2 Bytes) + Len(2 Bytes) + IMEI(15 Bytes) + | + Alarm Type(2 Bytes) + GPRMC + | + PDOP + | + HDOP + | + VDOP + | + Status(12 Bytes) + | + RTC(14 Bytes) + | + Voltage(8 Bytes) + | + ADC(8 Bytes) + | + LACCI(8 Bytes) + | + Temperature(4 Bytes) | +Mile-meter+| +Serial(4 Bytes) + | + Checksum (4 Byte) + \r\n(2 Bytes)

- Alarm type

  - 0x01  SOS button is pressed

  - 0x09  Auto Shutdown Alarm

  - 0x10  Low battery Alarm

  - 0x11  Over Speed Alarm

  - 0x13  Recover From Over Speed

  - 0x30  Parking Alarm

  - 0x42  Out Geo-fence alarm

  - 0x43  Into Geo-fence alarm

  - 0x50  INPUT-1 triggered (PIN 5)

  - 0x51  INPUT-1 not triggered (PIN 5)

  - 0x52  INPUT-2 triggered (PIN 7)

  - 0x53  INPUT-2 not triggered (PIN 7)

  - 0x54  INPUT-3 triggered (PIN 9)

  - 0x55  INPUT-3 not triggered (PIN 9)

  - 0x60  Begin Charge

  - 0x61  End Charge

  - 0x88  Heartbeat

  - 0x91  Into Sleep Mode

  - 0x92  Wakeup From Sleep Mode

  - 0xAA  Interval GPRS data

- Status(12 Bytes) —— Status：

  - Byte 01 —— SOS button

  - Byte 02 —— whether it connects to external power

  - Byte 03 —— reserved

  - Byte 04 —— reserved

  - Byte 05 —— Input 1 (PORT 5) for detecting engine on/off

  - Byte 06 —— Input 2 (PORT 7) for detecting the door open/close

  - Byte 07 —— Input 3 (PORT 9)

  - Byte 08 —— reserved

  - Byte 09 —— Digital output A  (PORT 6)

  - Byte 10 —— Digital output B  (PORT 8)

  - Byte 11 —— reserved

  - Byte 12 —— reserved

- Voltage(8 Bytes) ——Value of the voltage：

  - Format：ABBBIIII

  - A —— Charge Status (0 = Off Charge , 1 = On Charge)

  - BBB —— Battery Voltage (For example, 367 means 3.67V)

  - IIII —— Input Charge Voltage (For example, 1251 mean 12.51V)

- ADC(8 Bytes)  —— AD collection：

  - Format：CCCCDDDD

  - CCCC —— ADC0 collect (For example, 1251 means 12.51V)

  - DDDD —— ADC1 collect (For example, 1251 means 12.51V)（reserve）

- ADC Percent(6 Bytes)  —— AD collection：

  - Format:  AAABBB

  - AAA  ——  ADC0 collector percent( For example,056 means 56%)

  - BBB  ——  ADC1 collector percent( For example,056 means 56%)

- LACCI(8 Bytes) —— Location information elements：

  - Format：LLLLCCCC

  - LLLL ——  Location area code

  - CCCC ——  Cell ID

- Temperature(4 Bytes) —— Temperature（reserve for the device has no temperature sensor）：

  - Format：STTT

  - Precision is 0.1℃

  - The first byte “S” mean sign， such as“0/1/-”

  - Eg：0345 mean +34.5℃，1234 mean +123.4℃，-123 means -12.3℃

- Mile-meter(14 Bytes) —— Location information elements：

  - Format is AAAA.BBBBKm.

  - Four bytes after the radix point.

- Serial(4 Bytes) —— Serial number：

  - Format：SSSS

  - Every time reboot the device or reset, the serial number will initialize to 0001.

  - Every GPRS message send out will add one

  - After the serial number to 9999, restart from 0001 again.

The link of the explain about the CRC-16(modbus):

http://www.lammertbies.nl/comm/info/crc-calculation.html

Example of data:

$$B2359772032984289|AA$GPRMC,104446.000,A,5011.3944,N,01439.6637,E,0.00,,290212,,,A*7D|01.8|00.9|01.5|000000100000|20120229104446|14151221|00050000|046D085E|0000|0.0000|1170|29A7

$$:              The header of GPRS data

B2:              The length of this whole GPRS data, it’s in hex

359772032984289: It’s IMEI number of tracker

AA:              data alarm type

$GPRMC:        the header of GPRMC data

104446.000:       the time of tracker, means 13:21:58

A:               means tracker gets the GPRS signal

5011.3944        means the latitude, it’s 34 degree 37.4502 minute. It’s in Degree +

minute format

N:              means south latitude

01439.6637:     means the longitude, it’s 58 degree 25.7364 minute, it’s in degree+

minute format

E:              means west longitude

0.00:            speed

290212:         the date, means Feb 29th, 2012.

A*7D:           checksum for GPRMC

01.8:            PDOP

00.9:            HDOP

01.5:            VDOP

000000100000 :  the I/O status  value: 0 or 1

Byte 01 —— SOS button

Byte 02 ——whether it connects to external power

Byte 03 ——reserved

Byte 04 ——reserved

Byte 05 ——Input 1 (PORT 5) for detecting engine on/off

Byte 06 ——Input 2 (PORT 7) for detecting the door open/close

Byte 07 ——Input 3 (PORT 9)

Byte 08 ——reserved

Byte 09 ——Digital output A  (PORT 6)

Byte 10 ——Digital output B  (PORT 8)

Byte 11 ——reserved

Byte 12 ——reserved

20120229104446:    RTC time, means 10:44:46 Feb 29th, 2012

14151221:         1 :  means device is charged

415: means the internal battery is 4.15V

1221:  means external power is 12.21V

00050000:         ADA and ADB data , in other words: analogy data for fuel level

046D085E :        local GMS location code

0000:              temperature data

0.0000:            mileage data

1170:              serial number