Applied model:TK909,TK907,TKSTAR,TK-STARC8,TK808,GT06

Set up IP and port send SMS:  adminip123456 ip port

Such as:  adminip123456 220.231.142.241 7700

1.Upload V1 on-line information and data when the device or alarm

*XX,YYYYYYYYYY,V1,HHMMSS,S,latitude,D,longitude,G,speed,direction,DDMMYY,vehicle_status#

Including: * Command head

XX manufacturer names, such as: TH, DC, XY and so on.

Delimiter

YYYYYYYYYY automotive machine serial number.

CMD command center confirmed.

hhmmss time value is recognized in the command

HHMMSS automotive machine time, standard time, an 8-hour time difference with Beijing.

S: the data valid bit (A / V), A indicates that the GPS positioning data is valid data, V data indicates that the GPS positioning data is not valid.

latitude: degrees latitude (00 ~ 90), FF.FFFF:: Latitude, format DDFF.FFFF, DD latitude (00.0000 - 59.9999), retained four decimal places.

D: Latitude flag (N: latitude, S: latitude).

longitude: longitude format DDDFF.FFFF, DDD: degrees longitude (000 ~ 180), FF.FFFF:

Longitude (00.0000 - 59.9999), retained four decimal places.

G: longitude mark (E: E, W: W).

speed: speed, range of 000.00 - 999.99, to two decimal places.

The information field may be empty ie longitude, G,, direction,, indicates the speed is zero.

direction: azimuth, true north as 0 degrees, 1 degree resolution, clockwise.

The information field may be empty, such as longitude, G, speed,, MMDDYY,, represents the angle is 0.

DDMMYY: Day / Month / Year

vehicle_status: vehicle condition, a total of four bytes, which means that the state of automotive machine parts, vehicle parts status and alarm status. Hexadecimal values ​​represent ASCII characters, the following is the specific meaning of each bit of each byte of the variable, bit representation using negative logic that bit = 0 is valid. As follows:

| Rank | Retention |  | Automotive machine components status |  | State vehicle parts |  | Alarm status |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | First Byte |  | Second Byte |  | Third Byte |  | Fourth Byte |  |
| 0 | 0 |  | 0 |  | 0 |  | 0 |  |
| 1 | 0 |  | 0 | Keep | 0 |  | 0 | Panic |
| 2 | 0 | Panic button | 0 | Keep | 0 |  | 0 | Speed ​​alarm |
| 3 | 1 | Keep | 0 |  | 0 |  | 0 |  |
| 4 | 0 |  | 1 | Keep | 0 |  | 1 | Keep |
| 5 | 1 | Keep | 1 | Keep | 0 |  | 1 | Keep |
| 6 | 1 | Retention | 1 | Keep | 1 |  | 1 | Keep |
| 7 | 1 | Need not | 1 | Need not | 1 | Need not | 1 | Need not |

English Address Request Protocol (terminal requests):

* XX, YYYYYYYYYY, VI1, HHMMSS, S, latitude, D, longitude, G, speed, direction, DDMMYY, vehicle_status, net_mcc, net_mnc, net_lac, net_cellid #

English address returned protocol (server sends down):

* HQ, 4106000054, I1_2_EN, 130305,10,1,9, test12345

Chinese Address Request Protocol (terminal requests):

* XX, YYYYYYYYYY, V2, HHMMSS, S, latitude, D, longitude, G, speed, direction, DDMMYY, vehicle_status, net_mcc, net_mnc, net_lac, net_cellid #

Chinese address returned protocol (server sends down):

* HQ, 4106000054, I1, 130305,10,1,4, 5e 7f 4e 1c

net_mcc, net_mnc, net_lac, net_cellid # 460,01,43559,344224 #

# 2.Standard mode upload data to increase the base station location information:

Encoding format：

| No. | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 | 08 | 09 | 0A | 0B |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Content | $ | 0x1030731001 |  |  |  |  | 0x050316 |  |  | 0x220902 |  |  |
| Meaning | Recording Head | Automotive Machine Serial Number |  |  |  |  | Time |  |  | Date |  |  |

| No. | 0C | 0D | 0E | 0F | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Content | 0x22128745 |  |  |  | 0x00 | 0x113466574C |  |  |  |  | 0x014028 |  |  |
| Meaning | Latitude |  |  |  | Battery power display | Longitude values、N、E、AV |  |  |  |  | Speed, direction |  |  |

| No. | 19~1C | 1D | 1E | 1F | 20~23 | 24 | 25 | 26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Content | 0Xfffffbff | 0xff |  |  | 0x00001234 | 0x01CC |  | 0x01 |
| Meaning | vehicle_status | Usr_alarm_flag | GSM signals (1 to 31) | GPS signal | GPS units km mileage | Country code |  | Operators Number |

| No. | 27 | 28 |  | 29 | 2A | 2B |
| --- | --- | --- | --- | --- | --- | --- |
| Content | 0x8763 |  | 0x5B9C |  |  | 0x00 |
| Meaning | Station No. |  | Cell ID |  |  | Record number |

Description：“$”（0x24):Record Header,use for identifying records starting position.

Time:0x050316，Standard time: 5 point 3 minutes and 16 seconds,the same as Beijing Time:13 points 3 minutes and 16 seconds

Date:0x220902, September 22, 2002

Adopted latitude:0x22128745，22"12.8745

Adopted longitude:0x113466574C，113"46.6574

the last byte mean(0x15):

bit3, 1:East longitude,0:West longitude

bit2, 1:North latitude,0：Southern latitude

bit1，1：A，0：V

bit0，Not defined

Speed,direction:0x014028：Speed:014  direction:028.

vehicle_status、 Usr_alarm_flag：With binary representation of the vehicle state and user-defined alarm status。The same as SMS（represent ASCII characters)。

Record Number:Record the serial number of binary representation，plus"1"when send one Record.

GPS Mileage field, four-byte units kilometers

Eg: 0x00001234 indicates 0x1234 = 4660 Kilometers

Mileage Calibration: * HQ, 000, S32, 130305,1234 #

Initial calibration mileage: 0x1234 = 4660 Kilometers

Repay the data in the field is invalid

Base station information

Country code, 460 = 0x01CC

Operator number, 01 = 0x01

Station number, 34569 = 0x8763

Cell ID, 23452 = 0x5B9C

3.Base station information

*XX,YYYYYYYYYY,NBR,HHMMSS,MCC,MNC,TA,NUM,LAC,CID,RXLEV ,LAC,CID,RXLEV….,DDMMYY,vehicle_status#

XX：Manufacturer

YYYYYYYYYY：ID

HHMMSS Time

MCC  Country Code MCC(3-byte)

MNC  Network Code MNC(3-byte)

NUM  quantity of LBS，6-byte at most

TA    GSM Delay

LAC Location Area Code LAC(5-digit code)

CID Cell ID CID(5-digit code)

RXLEV Received Signal Level

DDMMYY Day / Month / Year

vehicle_status	（pls refer to Universal Data Difinition.）

E.g.：

*HQ,7893267560,NBR,081606,460,0,1,4,9338,3692,150,9338,3691,145,9338,3690,140, 9338,3692,139,220513,FFFFFBFF#

Multi-base battery Protocol: adding one byte behind status means battery.

*HQ,7893267560,NBR,081606,460,0,1,4,9338,3692,150,9338,3691,145,9338,3690,140, 9338,3692,139,220513,FFFFFBFF,cell#

cell range of value: 1-6

6=100%  5=80% 4=60% 3=20% 2=10%