GPRS Communication Protocol

This Protocol is used for communication between real time tracking platform server and the tracker device, only suit for CCTR-800/801/802/803/808/810 etc series GPS tracker and 999GPS platform, command & message from server to device is download command & message, command & message from device to server is upload command & message. The command & message is transfer by TCP/IP .

# Version

Version 1.2                 Date 2009-04-06

## 2.1 Symbol Defination

| Data Type | Description |
| --- | --- |
| CHAR | Signal ASCII word |
| C_STRING | ASCII word string, if the length not reach definition, add ASCII blank (0x20H) on the right. |
| N_STRING | ASCII digital string (0-9), if the length not reach definition, add ASCII zero (0x30H) on the left. |
| H_STRING | ASCII digital string (0-F), if the length not reach definition, add ASCII zero (0x30H) on the left. |
| HEX_STRING | ASCII HEX word string, for example 1 is 31, if the length not reach definition, add ASCII zero (0x30H) on the left. |
| BIN | Binary digital |
| BYTE | 8 bit integer without mark,0-255 |

## 2.2 Frame Format

The command & message between device and server is transferred with data frame format, the full format data frame definition is as follow:

| Frame head sign | Message serial number / time | Command word |
| --- | --- | --- |
| 1 byte | 12 byte | 4 byte |

Message contents

N byte (N is less than 2K)

Frame end sign

1 byte

## 2.3 Command & Message

## 2.3.1 Frame head & end sign

The sign is used to indicate the head and end of the frame,  frame start with 0X2AH(word “*”) as head sign, and frame end with 0X5EH(word “^”) as end sign, when transfer binary digital, need redefine the other mark sign byte, the server to device download message & command can not include : 0x2A(‘*’), 0x5E(‘^’).

Redefine:

2AH←─→7DH + 03H

5EH←─→7DH + 02H

7DH←─→7DH + 01H

### 2.3.2 Command List

Length: 4 byte, C_STRING type

Function: Define data frame command type, it indicate the command function:

Table 2 Command Defintion

| Command Type | Subclass | Serail number | Command Description | Remark |
| --- | --- | --- | --- | --- |
| D (Download message) | A | 00 | Alarm config message | Prealarm and alarm message |
|  |  | 01 | RED alarm message |  |
|  | B | 00 | Locate & check device | Device config |
|  |  | 01 | Shake hand message |  |
|  |  | 03 | Read device configuration |  |
|  |  | 04 | Read device running status |  |
|  |  | 06 | IP address |  |
|  |  | 07 | Center phone number |  |
|  |  | 11 | Set phone number |  |
|  |  | 12 | Set max speed |  |
|  |  | 14 | Set upload interval |  |
|  |  | 15 | Listen in |  |
|  |  | 17 | Read device phone number |  |
|  | C | 00 | Public message | Public communicate message |
|  |  | 01 | Dispatch Information |  |
|  |  | 02 | Reply call message(Taxi) |  |
|  |  | 03 | Call message (Taxi) |  |
|  |  | 04 | Navigation message |  |
|  | D | 00 | Set equitime upload | Locate message |
|  |  | 01 | Set equidistance upload |  |
|  | E | 00 | Reply yellow alarm upload message | Reply message |
|  |  | 01 | Reply red alarm upload message |  |
|  |  | 07 | Reply pick up passenger success (Taxi) |  |
|  | F |  |  |  |
|  | G |  |  |  |
|  | H | 00 | Circuit control signal | Control Signal |
|  |  | 01 | Oil supply control signal |  |
|  |  | 02 |  |  |
|  |  | 03 |  |  |
|  | X | 00 | Upload message reply message | Extended message |
|  |  | 01 | Alarm config message |  |
|  |  | 02 | Device function config command |  |
|  |  | 03 | Device mode config command |  |
|  |  | 04 | Inittial device command |  |
|  |  | 05 | Geo-fence config command |  |
|  |  | 06 | Device login platform reply message |  |
| U (Upload message) | A | 00 | Yellow alarm message | Prealarm and alarm message |
|  |  | 01 | Red alarm message |  |
|  | B | 00 | Shake hand message | Device status information |
|  |  | 02 | Reply read device config message |  |
|  |  | 03 | Reply read device running status message |  |
|  |  | 04 | Locate & Check message |  |
|  |  | 05 | Device logon information |  |
|  | C | 00 | Upload information | Commuication message |
|  |  | 07 | Pick up passenger success message (Taxi) |  |
|  | D | 00 | Continuous Equitime upload message | Device location information |
|  |  | 01 | Continuous Equidistance upload message |  |
|  |  | 02 | Continuous upload message end |  |
|  |  | 04 | Break point upload |  |
|  | E | 00 | Reply download set alarm config messag | Reply message |
|  |  | 01 | Reply download red alarm message |  |
|  |  | 04 | Reply dispatch message |  |
|  |  | 05 | Reply read phone number config |  |
|  |  | 06 | Reply phone number config |  |
|  |  | 08 | Reply set equitime upload message |  |
|  |  | 09 | Reply set equidistance upload message |  |
|  |  | 10 | Reply set upload interval when device is still |  |
|  |  | 20 | Reply accept call taxi message(Taxi ) |  |
|  |  | 21 | Reply call taxi message (Taxi) |  |
|  |  | 23 | Reply navigation message |  |
|  | F |  |  |  |
|  | G |  |  |  |
|  | H | 00 | Reply circuit control | Reply control signal |
|  |  | 01 | Reply oil supply control |  |
|  |  | 02 |  |  |
|  | X | 00 | General download message reply message | Extended message |

# Not defined serial command is used for future.

# DX00 & UX00 can be send at any time, and used to reply some burst out message.

### 2.3.3 Devce ID

Length : 15 byte, C_STRING type.

### 2.3.4 Message serial number / Time

Length : 12 byte, C_STRING type.

When server send reply message to device, this 12 byte contents is the message serial number, when the device send message to server is the same serial number, the other condition this 12 byte message is date and time message.

### 2.3.5 Message Content

Length: not fixed, <=1024 byte, and can be nop.

Funtion: define the device message of the command.

### 2.3.6 Parity Bit

Length：2 byte, HEX_STRING type, only used in SMS function.

Parity algorithm： All the message byte XNOR except head frame.

# 3. Command Description

## 3.1. Download command

### 3.1.1 Locate immediately

| Frame | Command word | Type | Length (Byte) | Description |
| --- | --- | --- | --- | --- |
| Head | * | CHAR | 1 |  |
| Serial / time |  | C_STRING | 12 |  |
| Command | DB00 | C_STRING | 4 |  |
| Device ID | Device ID | C_STRING | 15 |  |
| Message | Message | C_STRING | 3 |  |
| Message contents | LOG |  |  |  |
| Parity bit |  | HEX_STRING | 2 |  |
| End Symbol | ^ | CHAR | 1 |  |

For Example

*040331141830DB00LOG ^ Means download message date and time is :2004-03-31, 14:18.30, trigger device locate immediately and upload location immediately.

### 3.1.2 Shake Hand Message

| Frame | Command word | Type | Length (Byte) | Description |
| --- | --- | --- | --- | --- |
| Head | * | CHAR | 1 |  |
| Serial / time |  | C_STRING | 12 |  |
| Command | DB01 | C_STRING | 4 |  |
| Device ID | Device ID | C_STRING | 15 |  |
| Message | Message | C_STRING | 3 |  |
| Message contents | HSO |  |  |  |
| Parity bit |  | HEX_STRING | 2 |  |
| End Symbol | ^ | CHAR | 1 |  |

For Example

*040331141830DB01HSO ^ Means download message date and time is :2004-03-31, 14:18.30, download shake hand message to device.

### 3.1.3 Listen and Monitor sound in car

| Frame | Command word | Type | Length (Byte) | Description |
| --- | --- | --- | --- | --- |
| Head | * | CHAR | 1 |  |
| Serial / time |  | C_STRING | 12 |  |
| Command | DB15 | C_STRING | 4 |  |
| Device ID | Device ID | C_STRING | 15 |  |
| Message | Message | C_STRING | Not Fixed |  |
| Message contents | DB15XXX DB15 is fixed command XXXX is the telephone number used to listen the sound in car. |  |  |  |
| Parity bit |  | HEX_STRING | 2 |  |
| End Symbol | ^ | CHAR | 1 |  |

For Example

*040331141830DB1513322990033 ^ Means download message date and time is :2004-03-31, 14:18.30, Enable listen sound in car, and the telephone number is 13322990033.

### 3.1.4 Cut oil power supply (IO2)

| Frame | Command word | Type | Length (Byte) | Description |
| --- | --- | --- | --- | --- |
| Head | * | CHAR | 1 |  |
| Serial / time |  | C_STRING | 12 |  |
| Command | DH01 | C_STRING | 4 |  |
| Device ID | Device ID | C_STRING | 15 |  |
| Message | YY | C_STRING | 2 |  |
| Message contents | DH01YY DH01 is fixed command YY: control signal, 2 byte, fixed length. YY=00, means disconnect the oil power supply (IO2 is low output ,Engine will be cut off) YY=01, means connect the oil power supply (IO2 is high or open drain output , Engine will keep running) |  |  |  |
| Parity bit |  | HEX_STRING | 2 |  |
| End Symbol | ^ | CHAR | 1 |  |

For Example

*040331141830DH0101 ^ Means download message date and time is :2004-03-31, 14:18.30, connect the oil power supply (IO2 is high or open drain output , Engine will keep running).

### 3.1.5 General Upload Message Reply

| Frame | Command word | Type | Length (Byte) | Description |
| --- | --- | --- | --- | --- |
| Head | * | CHAR | 1 |  |
| Serial / time |  | C_STRING | 12 |  |
| Command | DX00 | C_STRING | 4 |  |
| Device ID | Device ID | C_STRING | 15 |  |
| Message |  | C_STRING | Not Fixed |  |
| Message contents | DX00 + upload command (4 byte) + operation result (1 byte) + error code (2 byte) + additional contents (different according the upload message) DH00 is fixed command Operation Result: 0: failed 1: success Error code: 00: unknown command. 01: parity error |  |  |  |
| Parity bit |  | HEX_STRING | 2 |  |
| End Symbol | ^ | CHAR | 1 |  |

### 3.1.6 Device login reply message

| Frame | Command word | Type | Length (Byte) | Description |
| --- | --- | --- | --- | --- |
| Head | * | CHAR | 1 |  |
| Serial / time |  | C_STRING | 12 |  |
| Command | DX06 | C_STRING | 4 |  |
| Device ID | Device ID | C_STRING | 15 |  |
| Message |  | C_STRING | 1 |  |
| Message contents | DX06 is fixed command 1: login success 0: login failed, device not exit. |  |  |  |
| Parity bit |  | HEX_STRING | 2 |  |
| End Symbol | ^ | CHAR | 1 |  |

## 3.2. Upload Command Description

### 3.2.1 Red alarm message

| Frame | Command word | Type | Length (Byte) | Description |
| --- | --- | --- | --- | --- |
| Head | * | CHAR | 1 |  |
| Serial / time |  | C_STRING | 12 |  |
| Command | UA01 | C_STRING | 4 |  |
| Device ID | Device ID | C_STRING | 15 |  |
| Message |  | C_STRING | 0 |  |
| Message contents | UA01XZ…Z+GPS Data UA01 is fixed command X: alarm code, 1 byte, HEX. Z…Z: Alarm content, ASCII word Alarm Code: 0: Device power supply disconnect (only send once) 1: There is accident (send every 3 seconds total 8 times until receive the reply message) 2: Car is hijack (send every 3 seconds total 8 times until receive the reply messages) 3. Car is alarming (send every 3 seconds total 8 times until receive the reply messages) |  |  |  |
| Parity bit |  | HEX_STRING | 2 |  |
| End Symbol | ^ | CHAR | 1 |  |

| For Example |  |
| --- | --- |
| *040331141830UA012Hi-jack061825A2934.0133N10627.2544E000.0040331309.6200000000L000000^ Means upload message date and time is :2004-03-31, 14:18.30, upload car is hi-jack alarm, alarm time is 14:18.25, and the locate GPS dta is : north 29 degree and 34.0133 minute, East 106 degree and 27.2544 minute, speed is 0, and direction is 309.62 . |  |
| Platform Reply command | DE01 |

### 3.2.2 Shake hand message

| Frame | Command word | Type | Length (Byte) | Description |
| --- | --- | --- | --- | --- |
| Head | * | CHAR | 1 |  |
| Serial / time |  | C_STRING | 12 |  |
| Command | UB00 | C_STRING | 4 |  |
| Device ID | Device ID | C_STRING | 15 |  |
| Message |  | C_STRING | 3 |  |
| Message contents | HSO |  |  |  |
| Parity bit |  | HEX_STRING | 2 |  |
| End Symbol | ^ | CHAR | 1 |  |

| For Example |  |
| --- | --- |
| *040331141830UB00HSO^ Means upload message date and time is :2004-03-31, 14:18.30, upload shake hand message. |  |
| Platform Reply command | DB01 |

### 3.2.3 Reply Locate immediately Message

| Frame | Command word | Type | Length (Byte) | Description |
| --- | --- | --- | --- | --- |
| Head | * | CHAR | 1 |  |
| Serial / time |  | C_STRING | 12 |  |
| Command | UB04 | C_STRING | 4 |  |
| Device ID | Device ID | C_STRING | 15 |  |
| Message |  | C_STRING | Not fixed |  |
| Message contents | UB04X…XY+GPS Data UB04 is fixed command. X…X: Device ID, 8 byte. Y: 1 byte, 0,1,2,3,4,5,6 0: No network or can not dial number. 1: Dial success, but can not connect SOCKET (Need follow current IP and Port) 2: No IP address, need set. 3: Voice status, can talking or send SMS. 4: Reply locate immediately once. 5: GPS error, GPS can not locate. 6: GPS data is not valid. |  |  |  |
| Parity bit |  | HEX_STRING | 2 |  |
| End Symbol | ^ | CHAR | 1 |  |

| For Example |  |
| --- | --- |
| *040331141830UB04999999984061825A2934.0133N 10627.2544E000.0040331309.6200000000L000000^ Means upload message date and time is :2004-03-31, 14:18.30, upload ID is CW9999C99999998, GPS data & time is 2004-03-31, 14:18.25. GPS is valid, and location is: north 29 degree and 34.0133 minute, East 106 degree and 27.2544 minute, speed is 0, and direction is 309.62 . |  |
| Platform Reply command | Not necessary reply |

### 3.2.4 Device login message

| Frame | Command word | Type | Length (Byte) | Description |
| --- | --- | --- | --- | --- |
| Head | * | CHAR | 1 |  |
| Serial / time |  | C_STRING | 12 |  |
| Command | UB05 | C_STRING | 4 |  |
| Device ID | Device ID | C_STRING | 15 |  |
| Message |  | C_STRING | Not fixed |  |
| Message contents | UB05+XX…X+GPS Data UB05 is fixed data XX…X: Device ID, 15 byte. |  |  |  |
| Parity bit |  | HEX_STRING | 2 |  |
| End Symbol | ^ | CHAR | 1 |  |

### 3.2.5 Upload Location Message Continuously

| Frame | Command word | Type | Length (Byte) | Description |
| --- | --- | --- | --- | --- |
| Head | * | CHAR | 1 |  |
| Serial / time |  | C_STRING | 12 |  |
| Command | UD00 | C_STRING | 4 |  |
| Device ID | Device ID | C_STRING | 15 |  |
| Message |  | C_STRING | Not fixed |  |
| Message contents | UD00+GPS Data UD00 is fixed data GPS Data refer 4 GPS data format. |  |  |  |
| Parity bit |  | HEX_STRING | 2 |  |
| End Symbol | ^ | CHAR | 1 |  |

### 3.2.6 Upload Location Message Continuously Finish

| Frame | Command word | Type | Length (Byte) | Description |
| --- | --- | --- | --- | --- |
| Head | * | CHAR | 1 |  |
| Serial / time |  | C_STRING | 12 |  |
| Command | UD02 | C_STRING | 4 |  |
| Device ID | Device ID | C_STRING | 15 |  |
| Message |  | C_STRING | Not fixed |  |
| Message contents | UD02+GPS Data UD02 is fixed data GPS Data refer 4 GPS data format. |  |  |  |
| Parity bit |  | HEX_STRING | 2 |  |
| End Symbol | ^ | CHAR | 1 |  |

### 3.2.7 Reply cut oil power supply

| Frame | Command word | Type | Length (Byte) | Description |
| --- | --- | --- | --- | --- |
| Head | * | CHAR | 1 |  |
| Serial / time |  | C_STRING | 12 |  |
| Command | UH01 | C_STRING | 4 |  |
| Device ID | Device ID | C_STRING | 15 |  |
| Message |  | C_STRING | Not fixed |  |
| Message contents | UH01+YY UH01 is fixed data YY is oil power supply status, 2 byte. 00: disconnect oil power supply, engine will be turned off. 01: connect oil power supply, engine will keep running. |  |  |  |
| Parity bit |  | HEX_STRING | 2 |  |
| End Symbol | ^ | CHAR | 1 |  |

| For Example |  |
| --- | --- |
| *040331141830 UH0100^ Means upload message date and time is :2004-03-31, 14:18.30, reply disconnect oil power supply message |  |
| Platform Reply command | Not necessary reply |

### 3.2.8 General Reply Download Message

| Frame | Command word | Type | Length (Byte) | Description |
| --- | --- | --- | --- | --- |
| Head | * | CHAR | 1 |  |
| Serial / time |  | C_STRING | 12 |  |
| Command | UX00 | C_STRING | 4 |  |
| Device ID | Device ID | C_STRING | 15 |  |
| Message |  | C_STRING | Not fixed |  |
| Message contents | UX00 + download command (4 byte) + operation result (1 byte) + error code (2 byte) + additional contents (different according the upload message , can be nop) UX00 is fixed command Operation Result: 0: failed 1: success Error code: 00: unknown command. 01: parity error |  |  |  |
| Parity bit |  | HEX_STRING | 2 |  |
| End Symbol | ^ | CHAR | 1 |  |

# 4. GPS data format

| Frame | Word | Type | Legth(Byte) | Description |
| --- | --- | --- | --- | --- |
| Time | HHmmSS | N_STRING | 6 | Hours / minutes / seconds |
| GPS valid or not |  | CHAR | 1 | A means GPS data is valid, V menas GPS data is not valid |
| Latitude |  | N_STRING | 9 | The first 2 byte unit is degree (0 - 90), and the follwing 7 byte unit is minute. |
| South or North | “N” or “S” | CHAR | 1 | N is north , and S is south |
| Longitude |  | N_STRING | 10 | The first 3 byte unit is degree (0 - 180), and the follwing 7 byte unit is minute. |
| East or West | “E” or “W” | CHAR | 1 | E is East, and W is West |
| Speed |  | N_STRING | 5 | Unit is Nautical Mile / hour |
| Date | YYMMDD | N_STRING | 6 | Year / month / day |
| Direction |  | N_STRING | 6 | 0-360, clockwise from north |
| IO State |  | C_STRING | 8 | 8 bit IO Power on off, ACC on away, Empty or Fully, Door open close. |
| Distance mark |  | CHAR | 1 | L means followed distance digit. |
| Distance |  |  | 6 | Distance digit, Unit is metre. |