GPS intelligent terminal

S508/528 PROTOCOL

Contents

I、Explain the concept of glossary	4

Packet head、Packet end	4

Packet length	4

Verify（XOR）	4

Pseudo Ip address	4

User ID	4

II、Center to terminals	4

Rollcall－30	4

View the terminal state－31	5

Single point send－32	5

Silence－33	5

Timing interval－34	5

Fixed distance interval－35	6

Network management alarm－36	6

Cancel the alarm－37	6

Restores the oil duct－38	7

Close the oil duct－39	7

Dispatch message－3A	7

Inquiry software version－3D	7

Telephone monitor－3E	8

Overspeed alarm－3F	8

Parking alarm－40	8

Power failure alarm－41	9

Download group phone－42	9

Download group phone content－43	9

Cancel the group telephone number limit－44	10

Camera instruction－55	10

Pronunciation instruction－58	11

Weary driving alarm setting－5F	11

Message rollcall－60	12

Cancel the alarm by message－61	12

Upgrade software－62	12

Software content download－63	12

Oil parameter－64	13

Long-distace reset－65	13

Message service number－66	13

Call up－67	14

LED screen protocol－68	14

LED screen open/close  68 - 01	14

LED signal deleting  68 - 02	15

LED message attribute 68 - 03	15

LED timing  68 - 04	16

LED lattice signal(only one color) 68 - 05	17

LED signal 68 - 06	17

Inquiry LED parameter 68 - 07	18

Inquiry LED signal content 68 - 08	18

Inquire LED screen type 68 - 09	19

Inquire LED screen state 68 – 0A	19

LED mode setting 68 – 0B	19

Function switch establishment—6A	20

Reset the mileage—6B	20

Auto photograph—6D	21

mileage revision－9F	21

III、Center to terminals	21

GPRS data echo instruction－21	21

IV Teminals to center	22

Heart beat packet function 21	22

Position data－80	22

The return of rollcall－81	24

Alarm data－82	25

The data of terminal’s status－83	26

Message to center－84	27

Terminal’s answer data－85	27

Version data－86	28

Picture transmission－8D	29

The blind area data of GPRS－8E	29

Refueling prompt－8F	30

Confirm of call up－93	30

The response of LED’s normal receive data－95	30

The return of LED’s property—9C	31

Retrun the mailbox’s content of the LED—9D	31

Return of the type and state of LED—9E	32

The transmission of the picture frame－54	32

The response of camera－56	33

Picture data－57	33

Appendix A  flowchart	34

AppendixB	35

# I、Explain the concept of glossary

## Packet head、Packet end

The packet head menas the beginning of the data stream.

The packet end means the end of the data sream.

## Packet length

Packet length means the length of the data content,It is from the next byte of the packet length until the packet end.

Explain:The packet length in some of the example of this protocol may be wrong,please according to this algorithm.

## Verify（XOR）

From the packet head of the data stream XOR with the next byte,until the previous byte of the calibration byte

## Pseudo Ip address

Using specfic algorithm change the SIM card number to Pseudo IP address,the algorithm in Appendix C

## User ID

The correct ASCII code base on the user name

# II、Center to terminals

## Rollcall－30

[Function] look over the current position of the designated vehicle.

[Direction] to the terminals

[Format]

| 24 | 24 | 30 | 00 | 06 |  |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo Ip Addr |  |  |  | calibration | Packet end |

## View the terminal state－31

[Function] 	look over the the working state of the designated vehicle

[Direction]	to the terminals

[Format]

| 24 | 24 | 31 | 00 | 06 |  |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | calibration | Packet end |

## Single point send－32

[Function] 	set the sampling method which assigns the terminals to send the position data to center

[Direction]	to the terminals

[format]

| 24 | 24 | 32 | 00 | 06 |  |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | calibration | Packet end |

## Silence－33

[Function] 	establish the terminals to stop the transmission data

[Direction]	to the terminals

[格式]

| 24 | 24 | 33 | 00 | 06 |  |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | calibration | Packet end |

## Timing interval－34

[Function] 	Modify the timing interval of the position data sending.

[Direction]	to the terminals

[Format]

| 24 | 24 | 34 | 00 | 0b |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | Parameter1 |  | Parameter 2 |  | Parameter 3 | calibration | Packet end |

[Instruction explanation]

Parameter1:double byte,hexadecimal form,means the ACC on time horizon(form 1 second to 18.2 hours).

Parameter2:double byte,hexadecimal form,means the ACC off time horizon(from 1 second to 18.2 hours).

Parameter3:single byte(choosing)when close the heartbeat functio,this instruction doesn’t include the parameter 3 the packet length is 0x0A,the heartbeat time is 31 seconds or 61 seconds or 91 seconds.

## Fixed distance interval－35

[Funcion] 	Modify the fixed distance interval of the position data sending.

[Direction]	to the terminals

[Format]

| 24 | 24 | 35 | 00 | 08 |  |  |  |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | parameter |  | calibration | Packet end |

[instruction explantion]

- parameter double byte, hexadecimal form(from 0m to 65535m)

- when the parameter is 0 means close the fixed distance interval function.

## Network management alarm－36

[Function] 	start alarm

[Direction]	to the terminals

[Format]

| 24 | 24 | 36 | 00 | 06 |  |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | calibration | Packet end |

## Cancel the alarm－37

[Function] 	cancel the alarm

[Direction]	to the terminals

[Format]

| 24 | 24 | 37 | 00 | 06 |  |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | calibration | Packet end |

## Restores the oil duct－38

[Function] 	restore the oil duct

[Direction]	to the terminals

[Format]

| 24 | 24 | 38 | 00 | 06 |  |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | calibration | Packet end |

## Close the oil duct－39

[Function] 	close the oil duct

[Direction]	to the terminals

[Format]

| 24 | 24 | 39 | 00 | 07 |  |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | calibration | Packet end |

## Dispatch message－3A

[Fucntion] 	dispatch message

[Direction]	to the terminals

[Format]

| 24 | 24 | 3A | 00 | 6+N |  |  |  |  | (N Byte) |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | Message content | calibration | Packet end |

## Inquiry software version－3D

[Function] 	inquiry the software release

[Direction]	to the terminals

[Format]

| 24 | 24 | 3D | 00 | 6 |  |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | calibration | Packet end |

## Telephone monitor－3E

[Function] 	telephone monitor

[Dircetion]	to the terminals

[Format]

| 24 | 24 | 3E | 00 | 6+X |  |  |  |  | X |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | Return number | calibration | Packet end |

[Instruction explantion]

- return number：compressed BCD code，the odd number makes up F,the even number makes up FF. Example：13812345678 is 13H 81H 23H 45H 67H 8FH  62664307 is 62H 66H 43H 07H FFH

- return number: not fixed-length.

- introduction: After on-hook, cancels the monitor automatically.

## Overspeed alarm－3F

[Fnction] 	set the overspeed alarm

[Direction]   to the terminals

[Format]

| 24 | 24 | 3F | 00 | 7 |  |  |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | Setting value | calibration | Pacetend |

[Instruction explantion]

- Setting value  is the upper limit threshold value of the overspeed alarm,when the setting value is 0,the overspeed alarm function is closed（form 0 to 255 km/hour）single byte, hexadecimal form.

## Parking alarm－40

[Function] 	set the parking alarm

[Direction]	to the terminals

[Formart]

| 24 | 24 | 40 | 00 | 7 |  |  |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | Setting value | calibration | Pacetend |

[Instruction explantion]

- Setting value is the upper limit threshold value of the parking time then the setting values is 0 the parking alarm function is closed（from 0 to 255minutes）single byte, hexadecimal form.

## Power failure alarm－41

[Function] 	set the power failure alarm

[Direction]	to the terminals

[Format]

| 24 | 24 | 41 | 00 | 7 |  |  |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | Setting value | calibration | Pacetend |

[Instruction explantion]

- Setting value  The setting value is 0 means the power failure alarm was closed,when the setting value si 1 means the power failure alarm was opend.(single byte hexadecimal form.)

## Download group phone－42

[Function] 	Download group phone number（the terminal returns 8C-01command）

[Direction]	to the terminals

[Format]

| 24 | 24 | 42 | 00 | 6 |  |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | calibration | Pacetend |

## Download group phone content－43

[Function] 	After the center receives the terminal’s request which to download the data packet,then use this instruction to download the data packet content.

[Direction]	to the terminals

[Format]

| 24 | 24 | 43 | 00 | 7+X |  |  |  |  |  | X |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | Packet serial number | Data content | calibration | Packet end |

[Instruction explantion]

- There are 50 number in every packet,when only one packet or the last packet,the packet serial number is FF,else the serial number is the terminal’s sending umber。

- Data content format：

| (1byte) | (8byte) | (1byte) | (20byte) |
| --- | --- | --- | --- |
| Contact person quantity | Contact person | Telephone quantity | Telephone number |

- If the contact person is not enough for the 8 byes,then add the blank space.When the telphone numbe not enough for the 20byte,then add 0.

- When have several contact person,then add the content behind it.

- Example:contact person is default code.eg于莹

  - 07 D3 DA D3 A8 D3 A8 00 00

- Example:telephone numer is ASC code,eg:15920061768 format is:

  - 0B 31 35 39 32 30 30 36 31 37 36 38 00 00 00 00 00 00 00 00 00  the first byte is total number which the telephone number occupy.

## Cancel the group telephone number limit－44

[Function] 	cancel the group telephone umber limit

[Direction]	to the terminals

[Format]

| 24 | 24 | 44 | 00 | 6 |  |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | calibration | Packet end |

## Camera instruction－55

[Function] 	camera instruction

[Direction]	camera device

[Format]

| 24 | 24 | 55 | 00 | 0A |  |  |  |  |  |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | command | paramerter1 | parameter2 | ID number | calibration | Packet end |

[Instruction explantion]

- command  camera control instruction,single byte, hexadecimal form。

- ID number This parameter（1-7）bit is the camera select.。

- Parameter 1 and parameter 2 is the attachment of command,the meaning of the parameter please see the table(single byte, hexadecimal form)

- Parameter 8 is baudrate，00 is 9600，01 is 19200

| command | parameter1 | parameter2 | explanation |
| --- | --- | --- | --- |
| 01H | 00H | Picture format（1-8） 1：the best 2：good 3：normal 4：bad | photograph |
| 02H | 00H | X:one frame data(0-x) | Read the assigned paicture data packet |
| 03H |  |  |  |
| 04H |  |  |  |
| 05H |  |  |  |
| 06H |  |  |  |
| 07H |  |  |  |
| 08H | 01 | 00H | Baudrate setting |
| 09H | 00H | 00H | Camera reset |

## Pronunciation instruction－58

[Function] 	give the instruction to the terminals to broadcast.

[Direction]	to the terminals

[Format]

| 24 | 24 | 58 | 00 | 07 |  |  |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | paramerter | calibration | Packet end |

[Instruction explanation]

- The parameter value is single byte（BCD code from 2 to 4）

| parameter | Meaning |
| --- | --- |
| 2 | Cross the border alarm |
| 3 | Weary driving |
| 4 | Front 200 m pronunciation |

## Weary driving alarm setting－5F

[Function] 	set the upper limit value of weary driving

[Direction]	to the terminals

[Format]

| 24 | 24 | 5F | 00 | 8 |  |  |  |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | Set value | Cusion value | calibration | Packet end |

[Instruction explantion]

- When the ACC on time reach to set value,the weary driving alarm on.the set value’s unit is hours

- Cusion value’s unit is minute

- When the set value is 0 the weary driving function is canceled

## Message rollcall－60

[Function] 	make use of message to rollcall

[Direction]	to the terminals

[Format]

| 24 | 24 | 60 | 00 | 06 |  |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packer length |  | Pseudo IP Addr |  |  |  | calibration | Packet end |

## Cancel the alarm by message－61

[Function] 	close the alarm by message

[Direction]	to the terminals

[Format]

| 24 | 24 | 61 | 00 | 06 |  |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packer length |  | Pseudo IP Addr |  |  |  | calibration | Packet end |

## Upgrade software－62

[Function] 	upgrade software at long-distance

[Direction]	to the terminals

[Format]

| 24 | 24 | 62 | 00 | 06 |  |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packer length |  | Pseudo IP Addr |  |  |  | calibration | Packet end |

## Software content download－63

[Function] 	send the software conent to the terminals

[Direction]	to the terminals

[Format]

| 24 | 24 | 63 | 00 | N |  |  |  |  | high | low | 256字节 |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packer length |  | Pseudo IP Addr |  |  |  | software address |  | Software content | calibration | Packet end |

## Oil parameter－64

[Function] 	examinate the oil changing

[Direction]	to the terminals

[Format]

| 24 | 24 | 64 | 00 | 0B |  |  |  |  |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packer length |  | Pseudo IP Addr |  |  |  | Parameter1 |  | direction | calibration | Packet end |

[instruction explanation]

parameter1:the range of the oil resistance changing

direction:1menas the null oil is the max resistance,2 means the full oil is the max resistance

## Long-distace reset－65

[Function] 	reset the terminals at long distance

[Direction]	to the terminals

[Format]

| 24 | 24 | 65 | 00 | 06 |  |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packer length |  | Pseudo IP Addr |  |  |  | calibration | Packet end |

## Message service number－66

[Function] 	set the terminal’s service mobile number

[Direction]	to the terminals

[Format]

| 24 | 24 | 66 | 00 | 19 |  |  |  |  | 0D |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packer length |  | Pseudo IP Addr |  |  |  | Data packet | calibration | Packet end |

Data packet format:”< mobile number>”   mobile:  11 byte,ASICC code beginning is ‘<’ symbole,the end is ‘>’ symbol.

## Call up－67

[Function] 	call up the terminals

[Direction]	to the terminal

[Format]

| 24 | 24 | 67 | 00 | 6 |  |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packer length |  | Pseudo IP Addr |  |  |  | calibration | Packet end |

## LED screen protocol－68

| 24 | 24 | 68 | 00 | 6+N |  |  |  |  | N | XOR | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packer length |  | Pseudo IP Addr |  |  |  | Slave order and parameters | calibration | Packet end |

[信令说明]

- Connect character M： If there is only one packet then set the M is 0.If there are several packets,Then the first packet’s M is 1,the second packet’s M is 2,the last packet’s M is the ASCII code of F.

- Except the slave order,all the parameters are Ascii code form.

### LED screen open/close  68 - 01

[Function]  open or close the LED screen

[Direction]	LED screen

| 01 | Data |
| --- | --- |
| Slave order | Display switch |

Data(ASCⅡ form):

0: open（30）

1: close（31）

example：send the close instruction to the LED screen of 12.34.56.78

24 24 68 00 07 12 34 56 78 01 31 XOR 0D

XOR is calibration

### LED signal deleting  68 - 02

[Function] 	control the signal of the LED screen

[Direction]	LED screen

| 02 | NNNN | MMMM |
| --- | --- | --- |
| Slave order | Beginning mailbox | End mailbox |

| 24 | 24 | 68 | 00 | 15 |  |  |  |  | 02 |  |  |  |  |  |  |  |  | XOR | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | Slave order | Beginning mailbox |  |  |  | End mailbox |  |  |  | calibration | Packet end |

NNNN,MMMM are 4 bytes ASCⅡ

example：delete the mailbox of the 0000-0010 and the terminal is 12.34.56.78

24 24 68 00 15 12 34 56 78 02 30 30 30 30 30 30 31 30 XOR 0D

XOR is calibration

### LED message attribute 68 - 03

[Fuction] 	set the LED message attribute

[Direction]	LED screen

[格式]

| 03 |  |  |  |  |  |  | Number; | Paracodes1,paras1;paracode2,paras2 | M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Slave order | mailbox |  |  |  | Space |  | Parameter’s number | parameter | Connect character |

[instruction explanation]

add a semicolon(;) between the parameter’s number and parameter,thre is no semicolon between the parameter and connect character.

ParaCode,Paras:

paraCode=A，paras=T1T2，T1=0/1/2，on behalf of open the LED in car moving、car stopping、car moving and stopping；T2=0/1/2/3/4/5/6/7/8，on behalf of left move or up move,see this table:

| Display mode | Left move | Up move | fixed | Opens from middle | Left cover | Right cover | Close from two side | blink | Down move |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Code | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |

paraCode=B，paras= font，font=1-16
paraCode=C，paras= disply speed，display speed =1-N，uint;byte/second
paraCode=D，paras= disply interval，interval=1-N，uint:second；
paraCode=E，paras= display numbers，display numbers =1-N；
paraCode=F，paras= display times，display times =1-N second；
paraCode=G，paras= brightnes levels，brightness =1-N；

paraCode=I，paras= time section，     
sectime=yymmddhhmmssyymmddhhmmssWk!yymmddhhmmssyymmddhhmmssWk!

note：there is must be a space between the mailbox number and parameter numbers,Wk means week (01 is monday，02 is tuesday，04 is wednesday，08 is thursday，03 means Monday and tuesday,00 means no limit)

| 24 | 24 | 68 | 00 | 13+M |  |  |  |  |  | 03 |  |  |  |  |  |  | ParaNum; |  | M | XOR | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | Salve order |  | Mailbox number |  |  |  | space |  | Parameter number | parameter | Connect character | calibration | Packet end |

Wk is ASC code,in hexadecimal form;

parameter：ParaCode1,paras1;ParaCode1,paras1;....;

example：$$ NNNN 2;A,20;B,2 00 XOR &

explanation：NNNN->mailbox number，there are 2 parameter sets，parameter A was set 0x20，parameter B was set 0x2,connect character is 01,there is a space between mailbox number and parameter number,from mailbox number to parameter,all is ASC code.

Much time sectin example:

- 2008-05-08 - 2008-10-08 everyday’s10:00 –12:00 display,14:00-18:00 display

$$ NNNN 3;A,20;B,2;I,08050810000008100812000000!08050814000008100818000000! 00 XOR

- everyday’s10:00-12:00 display

$$ NNNN 3;A,20;B,2;I,FFFFFF100000FFFFFF12000000! 00 XOR

- 2008-05-08 - 2008-10-08 display

$$ NNNN 3;A,20;B,2;I,080508FFFFFF081008FFFFFF00! 00 XOR

- 2008-05-13 – 2008-05-23 ,Display  in 10:00-14:00 at Monday and tuesday

$$ NNNN 3;A,20;B,2;I,08051310000008052314000003! 00 xor

### LED timing  68 - 04

[Function] 	modify the LED time

[Direction]	LED screen

[Format]

| 04 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Salve order | time |  |  |  |  |  |  |  |  |  |  |  | week |  |

[instruction explanation]

time，week are ASC code,Monday is 01，Tuesday is 02，Wednesday is 03 based on this to sunday

example:2008-1-1 12 :00:05 saturday : 30 38 30 31 30 31 31 32 30 30 30 35 36 ?? 0D

| 24 | 24 | 68 | 00 | 20 |  |  |  |  | 04 | 12byte |  | XOR | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | Slave order | time | week | calibration | Packet end |

example:

### LED lattice signal(only one color) 68 - 05

[Function] 	send LED lattice signal

[Direction]	LED screen

[Format]

| 05 |  |  |  |  |  | Data(N) | M |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Slave order | Mailbox number |  |  |  | Mailbox attribute | content | Connect character |

[instructon explanation]：

NNNN：mailbox number，from 0000——FFFF,the mailbox number is ASCII code。

Mailbox attribute:

| Mailbox attribute | Immediage message | Normal message | Company message | Public message | Alarm message |
| --- | --- | --- | --- | --- | --- |
| Code | 1 | 2 | 3 | 4 | 5 |

Data：the signal content（max is 512 byte）。

Lattice format: 192*32 pixel，in the identical line,every 8 pixels is one byte,when occupy the picture,take the first line of the 32 lines,then take the second,until the last line.

Res is 0,balck is 1

| 24 | 24 | 49 | 00 | 13+N |  |  |  |  | 05 |  |  |  |  |  | Data(N) |  | XOR | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | Slave order | Mailbox number |  |  |  | Mailbox attribute | Lattice content | Connetct character | calibration | Packet end |

### LED signal 68 - 06

[Function] 	send the LED signal

[Direction]	LED screen

[Format]

| 06 |  |  |  |  | A | data | M |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Slave order | Mailbox number |  |  |  | Mailbox attribute | Signal content | Connect character |

[instruction explanation]：

NNNN：mailbox number，form 0000——FFFF the form is ASCII code。

Mailbox attribute:

| Mailbox attribute | Immediate message | Normal message | Company message | Public message | Alarm message |
| --- | --- | --- | --- | --- | --- |
| code | 1 | 2 | 3 | 4 | 5 |

Data：the signal content

| 24 | 24 | 68 | 00 | 13+N |  |  |  |  | 06 |  |  |  |  | A | data | M | XOR | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | Slave order | Mailbox number |  |  |  | Mailbox attribute | Signal content | Connect character | calibration | Packet end |

example:

### Inquiry LED parameter 68 - 07

[Function] 	inquiry the LED parameter

[Direction]	LED screen

[Format]

| 07 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Slave order | mailbox |  |  |  |

NNNN：ASCII form。

| 24 | 24 | 68 | 00 | 11 |  |  |  |  | 07 |  |  |  |  | XOR | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packer head |  | Main order | Pakcet length |  | Pseudo IP Addr |  |  |  | Slave order | Mailbox number |  |  |  | calibration | Packet end |

### Inquiry LED signal content 68 - 08

[Function] 	Inquiry LED signal content

[Direction]	LED screen

[Format]

| 08 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Slave order | Mailbox number |  |  |  |

NNNN:ASCII form

| 24 | 24 | 68 | 00 | 11 |  |  |  |  |  | 08 |  |  |  |  | XOR | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pakcet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | Slave order |  | Mailbox |  |  |  | calibration | Packet end |

### Inquire LED screen type 68 - 09

[Function] 	inquire the LED type

[Direction]	LED screen

[Format]

| 24 | 24 | 68 | 00 | 07 |  |  |  |  | 09 | XOR | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | Slaver order | calibration | Packet end |

### Inquire LED screen state 68 – 0A

[Function] 	Inquire LED screen state

[Direction]	LED screeen

[Format]

| 24 | 24 | 68 | 00 | 07 |  |  |  |  | 0A | XOR | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | Slaver order | calibration | Packet end |

### LED mode setting 68 – 0B

[Function] 	enable or disable the LED mode

[Direction]	LED

[Format]

| 0B | Data |
| --- | --- |
| Slave order | mode |

Data(ASCⅡ code form):

1: LED screen（31）

2: handle mode （32）

example：send the LED instruction to 12.34.56.78

24 24 68 00 07 12 34 56 78 0B 31 ?? 0D

## Function switch establishment—6A

[Function] 	enable or disable the function

[Direction]	to the terminals

| 24 | 24 | 69 | 00 | 09 |  |  |  |  | Data1 | Data2 | Data3 | XOR | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | parameter |  |  | calibration | Packet end |

- Ddata1: emergency alarm switch：1 is opend，0 is closed

- Data2:  1 byte

Bit0=1, air conditoning examination is opened, =0 is closed    default closed

Bit1=1  low voltage examination is opened ,=0 is closed   default closed

Bit2=1  door signal examination is opened, =0 is closed  default closed

Bit3=1  undervolatge alarm is opened，=0 is closed  default closed

Bit4=1  overvolatge alarm is opened,=0 is closed  default closed

Bit5=1  ACC examinatin is opened, =0 is closed  default closed

Bit6=1  oil change alarm is opend,=0 is closed  default closed

Bit7=1,  ACC related data,default opend

- Data3:1 byte

Bit0=0, reserved

Bit1=0, reserved

Bit2=0, reserved

Bit3=0, reserved

Bit4=0, reserved

Bit5=0, reserved

Bit6=0, reserved

## Reset the mileage—6B

[Function] 	reset the mileage

[Direction]	to the terminals

[Format]

| 24 | 24 | 6B | 00 | 06 |  |  |  |  |  |  | XOR | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | parameter |  | calibration | Packet end |

[instruction explanation]

parameter：the value of the mileage(unit:km)

## Auto photograph—6D

[Function] 	auto photograph

[Direction]	to the terminals

[Format]

[Instruction explanation]

| 24 | 24 | 6D | 00 | 0B |  |  |  |  | h | l | h | l |  | XOR | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | Start time |  | End time |  | interval | calibration | Packet end |

The h of the start time and end time on behalf of hours,the l of the start time and end time on behalf of minutes

If the interval is 0 on behalf of the auto phontograph is closed

The interval time’s unit is minutes

example：from ９：００ to １７：００，send a picture every 10 minutes.

24 24 6D 00 0B C A2 32 98 09 00 17 00 10 ?? 0D

## mileage revision－9F

[Function] 	revise the distance of the two unlocated position

[Direction]	to the terminals

[Format]

| 24 | 24 | 9F | 00 | N |  |  |  |  | data | data | XOR | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | Slave order | data | calibration | Packet end |

Slave order=01:  return the straight distance between the 2 point.(uint:m)

Example:the distance is 240m  data= 22 32 34 30 22 the data in the double quote

Slave order =02   open the revision function,there is no data in it

Slave order =03   close the revision function,there is no data in it.

# III、Center to terminals

## GPRS data echo instruction－21

[Function]	The center send the “link confirm instruction” to the terminals when received the linkage information of the vehicle。

[Direction]	to the terminals

[Format]

| 24 | 24 | 21 | 00 | 05 |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Calibration value | Main order | Slave order | calibration | Packet end |

[Instruction explanation]

- Calibration value: It is the value of the return data of the vehicle.(1 byte)

- Main order:It is the main order of the return data of the vehicle (1 byte);

Slave order:It is the slave order of the return data of the vehicle (1 byte);

# IV Teminals to center

## Heart beat packet function 21

[Function]	The terminals send connecting information to center.

[Direction]	center

[Format]

| 24 | 24 | 21 | 00 | 06 |  |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | calibration | Packet end |

## Position data－80

[Function]	The terminals send the position data to center.

[Direction]	center

[Data should be putted in to database] When the center receives the data then saves it into the dynamic position data table（DynData）

[Format]

| 24 | 24 | 80 | 00 | 23 |  |  |  |  | （24Byte） |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | Position data | calibration | Packet end |

[instruction explanation]

- Position data form

| 04 | 12 | 12 | 08 | 12 | 00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| time |  |  |  |  |  | latitude |  |  |  | longitude |  |  |  |  | speed |  | Direction angle |  | position | No-load | key switch | A/D data |  |  |  | mileage |  |  |  |  |  |

Times form：

Date range：the latter two bit of the year ；month，1-12；day，1-31

Example:2009/10/3 09 10 03

Time range：hour，00——23；minute，00——59；second，00——59

hour、minute、second is BCD。

[example]：	ten minutes to eight：

07H，50H，00H

latitude form：

latitude range：00.000 minute 0 degree——59.999 minutes 89 degrees ；

Data form is BCD form,but the highest bit is sign bit。“positive”means“north latitude”，“negative”means“south latitude”；

The unit of “minute” is：1/1000 minute。

[example]：	Latitude 901 Minutes 30 Degrees south：

3H，03H，79H，01H

longitude：

longitude form：00.000 minute 0 degree——59.999 minutes 179 degrees；

Data form is BCD form,but the highest bit is sign bit。“positive”means“east longitude”，“negative”means“west longitude”；

The unit of “minute” is：1/1000 minute。

[example]：	Longitude 45.608 Minutes 130 Degrees west

13H，04H，56H，08H

speed form：

speed range：0——9999km/hour

data form is BCD form。

[example]：	120km/hour：

01H，20H

direction angle form：

rang：000——359 degree

data form is BCD form, Due north is 0 degrees, clockwise counts。

unit：degree。

[example]：	154 degree：

01H，54H

position status：

position status is a single byte：

| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| D7 | Mark flag |
| --- | --- |
| 0 | delocalization |
| 1 | position |

| D6 | Differential positioning |
| --- | --- |
| 0 | no |
| 1 | yes |

| D5 | D4 | introduction |
| --- | --- | --- |
| X | X | reserved |

| D3 | D2 | D1 | D0 | satellite |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | range：0—15 |
| 1 | 1 | 1 | 1 |  |

No load or full load status：

：

| D7 |  |
| --- | --- |
| D6 | 1：deault system use |
| D5 | No load or full load examination（1：full 0：no） |
| D4 | Antenna short(0,normal,<> 0 short) |
| D3 | Antenna open(0,normal,<> 0 open) |
| D2 | Air conditioning examination（0：open 1：close） |
| D1 | Break-in examination（0：open 1：close） |
| D0 | Low voltage examination（0：open 1：close） |

Key switch:

1：key opend  0：key closed

AD data (double byte Hexadecimal form )

The first 2 bytes are oil resistance,the latter two are voltage value.

example：50 ohm is 01F4

example:0000H is 0 V;    0B3AH is 11.58 V

Voltage conversion step:

- The 16 bits of the first two byte were changed to the interger of the voltage.

- The last two bytes were change to decimal.

mileage：

The current mileage of the vehicle.unit:m/s

## The return of rollcall－81

[Function]	The return of the rollcall

[Direction]	center

[The data should be inputed into database] （DynData）

[Format]

| 24 | 24 | 81 | 00 | 23 |  |  |  |  | （24Byte） |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | Position data | calibration | Packet end |

## Alarm data－82

[Function]	Terminal send the alarm data initiatively to center

[Direction]	center

[The data should be inputed into database] （DynData）

[Format]

| 24 | 24 | 82 | 00 | 25 |  |  |  |  | （24Byte） |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | Position data | Alarm data |  | calibration | Packet end |

[Instruction explanation]

Alarm data(dobule Hexadecimal byte)

The first byte

| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| D7 |  |
| --- | --- |
| D6 | oil change alarm |
| D5 | Cross the border alarm |
| D4 | Overvoltage alarm |
| D3 | Undervoltage alarm |
| D2 | The people of overload alarm |
| D1 | Overtime driving alarm |
| D0 | Enter in to the border alarm |

The second alarm data

| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| D7 | The alarm of the door was opened illegally |
| --- | --- |
| D6 | Start the vehilce illegally |
| D5 | Vibration alarm |
| D4 | Center enable the terminal to alarm |
| D3 | Power failure alarm |
| D2 | Parking alarm |
| D1 | Overspeed alarm |
| D0 | Emergency alarm |

1:alarm

0:no alarm

## The data of terminal’s status－83

[Function]	The return of the instruction of 31

[Dircetion]	to center

[Format]

| 24 | 24 | 83 | 00 | 1E |  |  |  |  | （24Byte） |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | Stauts data | calibration | Packet end |

[instruction explanation]

- Format of stauts data

| sampling time |  |  |  |  |  | Alarm status |  |  | Whether to locate |  | Sampling type | Sampling value |  | Sending type | Car stop setting | Overspeed setting | Phone limit | Area node limit | Safe setting | Long time driving | AA |  | BB | CC | 保留 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

AA:sampling value(acc off)

BB:emergency alarm switch

CC:The related of photograph

| name | length | description |
| --- | --- | --- |
| Sampling time | 6 | BCD code |
| Alarm stauts | 2 | Double Hexadecimal byte |
| Whether to locate | 1 | 1 yes 0 no |
| Sampling type | 1 | 1 fixed time 0 fixed distance |
| Sampling value | 2 | Double Hexadecimal byte |
| Sending type | 1 | 1 point send 2 silence |
| Car stop setting | 1 | 0 is no setting the others is the correct value.single byte( Hexadecimal) |
| Overspeed setting | 1 | 0 is no setting ther others is the correct value.single byte(Hexadecimal) |
| Phone limit | 1 | 1 group phone limit 0 no limited |
| Area node limit | 1 | 1 limit 0 no limit |
| Safe setting | 1 |  |
| long time driving(two bytes) | 2 | Long time driving |
| Sampling value(acc off) | 2 | Double byte(Hexadecimal) |
| Emergency alarm switch | 1 | 1opened，0 closed |
| The related of photograph | 1 |  |
| reserved | 1 | reserved |

## Message to center－84

[Function]  The terminal send message to center

[Direction]	To center

[The data shouldn’t to be inputted in to database]

[Format]

| 24 | 24 | 84 | 00 | 6+N |  |  |  |  | （N Byte） |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | Short message | calibration | Packet end |

[Instruction explantion]

- The length of short message: The max length is 160 byte

## Terminal’s answer data－85

[Function]	The terminal send the corect data when receive the center’s instruction

[Direction]	To center

[The data shouldn’t to be inputted in to database]

[Format]

| 24 | 24 | 85 | 00 | 0B |  |  |  |  | （5Byte） |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | Answer data | calibration | Packet end |

[Instruction explantion]

- The format of return data

| Main order | Slave order | Success or failure | reserved |  |
| --- | --- | --- | --- | --- |

- Main order (single byte)( Hexadecimal)

- Slave order (single byte)( Hexadecimal)

- Success or failure   BCD code  1 success 0 failure

## Version data－86

[Function]	the return data of the software inquiry(the instruction of 3d)

[Direction]	to center

[The data shouldn’t to be inputted in to database]

[Format]

| 24 | 24 | 86 | 00 | 0E |  |  |  |  | （8Byte） |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | Version data | calibration | Packet end |

[Instruction explanation]

- Version data format（8bytes）

| type | 00~FF | eg |
| --- | --- | --- |
| Software type | 00~FF | 001 |
| year |  | 04 |
| month |  | 12 |
| date |  | 15 |
| clock |  | 12 |
| Function 1 |  |  |
| Function 2 |  |  |

- type

| 00 | 30 |
| --- | --- |
| 20 | 32 |
| a0 | 33a |
| 80 | M508 |
| 81 | M518 |
| 82 |  |
| 83 |  |
| 84 |  |
| 85 |  |
| 86 |  |
| 87 |  |

- Function 1

| D7 | support camera function |
| --- | --- |
| D6 | Support Long-distance software download |
| D5 | Support car watch |
| D4 | Support safe setting |
| D3 | Support emergency alarm function |
| D2 | support phone function |
| D1 | The LCD screen can be connected |
| D0 |  |

- Function 2

| D7 |  |
| --- | --- |
| D6 | support pronunciation function |
| D5 | The protocol isn’t our company |
| D4 | Support double card |
| D3 | Suport scanning function |
| D2 | Support voice alarm fucntion |
| D1 | Support LED screen |
| D0 | Supoort A/D |

## Picture transmission－8D

[Function]	The terminal send the picture data to center when received the instruction of photograph

[Direction]	To center

[The data shouldn’t to be inputted in to database]

[Format]

| 24 | 24 | 8D | 00 | N+8 |  |  |  |  |  |  | N字节 |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | The serial number of picture | The serial number of packet | Picture data | calibration | Packet end |

[Instruction explanation]

- The serial number of picture:Because of the picture was transmited by packet,so the this byte is same in the same picture

- The serial number of packet: The Nth packet of the picture

- Picture data:The content of the picture

## The blind area data of GPRS－8E

[Function]	When the GPRS signal is not connected,the position data was stored,when the GPRS signal is connecte,the terminal send the stored data to the center.

[Direction]	To center

[The data should be inputted in to database]

[Format]

| 24 | 24 | 8E | 00 | 23 |  |  |  |  | （24Byte） |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | Position data | calibration | Packet end |

## Refueling prompt－8F

[Function]	When the oil was increased,the terminal send this instruction to center

[Direction]	To center

[The data should be inputted in to database]

[Format]

| 24 | 24 | 8F | 00 | 08 |  |  |  |  | （24Byte） |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pakcet head |  | Main order | Pakcet length |  | Pseudo IP Addr |  |  |  | Position data | calibration | Packet end |

## Confirm of call up－93

[Function]	The return of call up

[Direction]	To center

[The data should be inputted in to database]

[Format]

| 24 | 24 | 93 | 00 | 6 |  |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | calibration | Packet end |

## The response of LED’s normal receive data－95

[Function]	To center by terminal

[Direction]	To center

[Format]

| 24 | 24 | 95 | 00 | 14 |  |  |  |  | Return value |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pakcet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | KKKKLLLL | calibration | Packet end |

[Instruction explanation]

- The return value are 8 ASCII code，The 4 bytes of KKKK represent the LED’s manufacturer，the 4 bytes of LLLL represent the version of device。

## The return of LED’s property—9C

[Function]	The terminal send this instruction to center when receive the parameter of the maibox of the LED

[Direction]	To center

[Format]

| 24 | 24 | 9C | 00 | 10+N+M |  |  |  |  |  |  |  |  |  |  | ParaNum; | ParaCode1,paras1;ParaCode1,paras1;.... | M | XOR | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | Mail box number |  |  |  |  | Parameter’s number |  | parameter | Connect character | calibration | Packet end |

Instruction explanation

There is a semicolon between the parameter’s number and parameter.There is no semicolon between the parameter and connect character

## Retrun the mailbox’s content of the LED—9D

[Direction]	to center

| 24 | 24 | 9D | 00 | 12+N |  |  |  |  |  |  |  |  |  |  | A | S | data | M | XOR | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | The serial number of mailbox |  |  |  |  | Property of maibox |  | Property of content | Signal content | Connect character | calibration | Packet end |

[Instruction explanation]：

A：The attribute of the mailbox

| attribute | Immediate message | Normal message | Company message | Public message | Alarm message |
| --- | --- | --- | --- | --- | --- |
| code | 1 | 2 | 3 | 4 | 5 |

Data：The content of the mailbox。

M：same to the M of the 68 instruction

S:01->return the signal content,02->return the lattice content

## Return of the type and state of LED—9E

[Function] 	inquiry the type and state of the LED

[direction]	   to center

[format]

| [instruction explantion] 24 | 24 | 9D | 00 | 08 |  |  |  |  | data | data | XOR | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | Slave order | data | calibration | Packet end |

Slave order=01 the type of the LED

Data=01  32*192

Data=02  16*128

Slve order=02 the state of the LED

Data=01  the state is good

Data=02,

Revision of mileage—9F

[Function] 	Revise the distance of the two not target point

[Direction]	to center

[Format]

| 24 | 24 | 9F | 00 | 17 |  |  |  |  | data | data | XOR | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | Slave order | data | calibration | Packet end |

Slave order=00:  The content is the longitude and latitude of the two point(The center should calcuate the distance of the two point and send the result to terminal)

Data content:

Long1+lat1+long2+lat2   The every data is 4 bytes and the high byte is in front of them

example:  a(114.02396,22.31182) b(114.02420,2232203)

24 24 9F 00 17 0C A2 32 A1 (packet head)

00 11 40 23 96 02 23 11 82 11 40 24 20 02 23 22 03

B5 0D

## The transmission of the picture frame－54

[Function]	When the picture data were losted or center didn’t receive them,the center send this instruction to terminal,and the terminal send this frmae again.

[Direction]	To terminal

[Format]

| 24 | 24 | 54 | 00 | 08 |  |  |  |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | Serial number of frame | reserved | calibration | Packet end |

## The response of camera－56

[Function] 	the response of camrea instruction

[Direction]	to center

[Format]

| 24 | 24 | 56 | 00 | 07 |  |  |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | response | calibration | Packet end |

[Instruction explanation]

- Response:

| response | introduction |
| --- | --- |
| 01H+total frame number | Photo success，is saving data |
| 02H | MCU receive all the picture data |
| 03H |  |
| 04H |  |
| 05H |  |
| 06H |  |
| 07H | Test ok |
| 09H | Reset success |

## Picture data－57

[Function] 	picture data upload

[Direction]	To center

[Format]

| 24 | 24 | 57 | 00 | 519 |  |  |  |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet head |  | Main order | Packet length |  | Pseudo IP Addr |  |  |  | frame | data | calibration | Packet end |

[instruction explanation]

- frame   the current frame of the picture data。

- data    the data of this picture frame。

# Appendix A  flowchart

Client  request

connection

Center authenticate

Center answer to client when receive the request

Password error,close connetion

Client send the data of authentication to center

password ok,begin service

Client’s down instruction

AppendixB

Rturn to client is the send fail

Center send this instruction

Terminal

Appendix C How to change the SIM cand number to pseudo IP addr

Method：

- Cut off the first bit of the SIM card number

Example 13512345006 ，cut off the first bit is 3512345006

- Get 5 froup number from the 10 number

Example: get 35 12 34 50 06 from 3512345006

- Take the latter four numbers of this group,and change them in to binary form

Example:

  - The latter four numbers of group of 35 12 34 50 06 is 12 34 50 06

  - The binary of 12 is 00001100

  - The binary of 34 is 00100010

  - The binary of 50 is 00110010

  - The binary of 06 is 00000110

  - So the group number is 01100  00100010  00110010  00000110

- Get the first froup number in second step and subtract 30,then change it into binary,at last add it in to the four number separately which from the third step

  - The first number of 35 12 34 50 06 is 35

  - 35 – 30 = 5

  - The binary of 5 is 0101

  - Use the first number of third step,then adds 0 on the top digit(0 is the first bit of the 5 which is binary),at last get 00001100, change 00001100 to decimal is 12.

  - Use the second number of third step,then adds 1 on the top digit(1 is the second bit of the 5 which is binary),at last get 10100010, change 10100010 to decimal is 162.

  - Use the third number of third step,then adds 0 on the top digit(0 is the third bit of the 5 which is binary),at last get 00110010, change 00110010 to decimal is 50

  - Use the fourth number of third step,then adds 1 on the top digit(1 is the fourth bit of the 5 which is binary),at last get 10000110, change 10000110 to decimal is 134

- Get the pseudo IP addr

The pseudo IP addr of 13512345006 is 12.162.50.134