GPS&GSM/GPRS

Vehicle Tracker Protocol

(V1.03)

This GPRS Protocol is for Uniguard GPS Tracker UM02 and UT04

The protocol is applied for our GPS vehicle tracker products.

It adopts UDP connection to connect Tracker and GPRS_SERVER，there are 2 types:

1. TRACKER transmits to GPRS_SERVER, message format is：Message head + message body,

GPRS_SERVER transmits to TRACKER，message format is：Message head + message body +message end signal

# Tracker GPRS_Server

## Message Format

Tracker send to GPRS_Server message：Message_Head+Message_body

| No. | Field | TYPE | Length | description |
| --- | --- | --- | --- | --- |
| 1 | packetLen | short | 2 | Message length |
| 2 | CMD | short | 2 | Command ID：0x0008 upload position 0x8009 control response 0x0003 Alarm message 0x0000 Shake hand 0x0200 image size 0x0201 image packet |
| 3 | MsgBody |  |  |  |

## Upload position Message body

CMD_PosReq			0x0008

| No. | Field | TYPE | Length | description |
| --- | --- | --- | --- | --- |
| 1 | bEnable | unsigned char | 1 | Status: 0bit is locating 1bit Main power ;2,3,4,5bit is NULL; 6bit ACC status ; 7bit poff |
| 2 | bAlarm | unsigned char | 1 | Default 0x80 |
| 3 | nSpeed | char | 1 | Speed |
| 4 | nDirection | short | 2 | Direction angle |
| 5 | fLongitude | double | 4 | Longitude |
| 6 | fLatitude | double | 4 | Latitude |
| 7 | nDateTime | long | 4 | Date |
| 8 | sUserID | char | 11 | Device ID |
| 9 | nIOState | char | 1 | I/O status |
| 10 | nOilState | char | 1 | Value of Fuel |

## Response of control Message body

CMD_CommandResp		0x8009

| No. | Field | TYPE | Length | description |
| --- | --- | --- | --- | --- |
| 1 | nGisIp | int | 4 | GIS User IP |
| 2 | nPort | Unsigned int | 4 | GIS User ID |
| 3 | bEnable | unsigned char | 1 | Status 0 is locating ; 1bit main power; 2,3,4,5bit NULL; 6bit ACC status; 7pOff |
| 4 | bAlarm | unsigned char | 1 | 0x80; |
| 5 | nSpeed | char | 1 | Speed |
| 6 | nDirection | short | 2 | Direction angle |
| 7 | fLongitude | double | 4 | Longitude |
| 8 | fLatitude | double | 4 | Latitude |
| 9 | nDateTime | long | 4 | Date |
| 10 | sUserID | char | 11 | Device ID |
| 11 | nIOState | char | 1 | I/O status |
| 12 | nOilState | char | 1 | Value of fuel |
| 13 | cErrorCode | Char | 1 | 1-OK,0-failed |

## Upload the alarm Message body

CMD_AlarmReq			0x0003

| No. | Field | TYPE | Length | description |
| --- | --- | --- | --- | --- |
| 1 | bEnable | unsigned char | 1 | Status: 0bit is locating 1bit Main power ;2,3,4,5bit is NULL; 6bit ACC status ; 7bit poff |
| 2 | bAlarm | unsigned char | 1 | (0x80)1:='SOS Alarm'; 2:='over_speed alarm'; 3:='e_fence alarm'; 9:=’Cut Main power’ |
| 3 | nSpeed | char | 1 | Speed |
| 4 | nDirection | short | 2 | Direction angle |
| 5 | fLongitude | double | 4 | Longitude |
| 6 | fLatitude | double | 4 | Latitude |
| 7 | nDateTime | long | 4 | Date |
| 8 | sUserID | char | 11 | Device ID |
| 9 | nIOState | char | 1 | I/Ostatus |
| 10 | nOilState | char | 1 | Value of fuel |

## Shake_hand Message_body

CMD_ShakeHandReq	0x0000

| No. | Field | TYPE | Length | description |
| --- | --- | --- | --- | --- |
| 1 | sUserID | char | 11 | Device ID |

## Upload image

CMD_UploadFileReq	0x0200

| No. | Field | TYPE | Length | description |
| --- | --- | --- | --- | --- |
| 1 | sUserID | char | 11 | Device ID |
| 2 | nDate | long | 4 | date |
| 3 | nTime | long | 4 | time |
| 4 | nFileLen | unsigned short | 2 | Size of image |

## Packet of image (TRACKER GPRS_SERVER)

CMD_SendJpgReq	0x0201

| No. | Field | TYPE | Length | description |
| --- | --- | --- | --- | --- |
| 1 | sUserID | char | 11 | Device ID |
| 2 | nPage | unsigned short | 2 | Number of packet |
| 3 | nPageLen | unsigned short | 2 | Packet length |
| 4 | sData | char | 506 | Image data |

# GPRS_Server  TRACKER

## Message format

GPRS_Server send to Tracker format：message_head+message_body+message_end

| No. | Field | TYPE | Length | description |
| --- | --- | --- | --- | --- |
| 1 | sMark | char | 6 | Flag of message (\r\n*KW) |
| 2 | packetLen | short | 2 | Message Length |
| 3 | CMD | short | 2 | Command ID |
| 4 | MsgBody |  |  |  |
| 5 | sEnd | char | 2 | message end "\r\n" |

## Response of Shake hand

CMD_ShakeHandResp	0x8000

| No. | Field | TYPE | Length | description |
| --- | --- | --- | --- | --- |
| 1 | cErrorCode | char | 1 | status |

## Response of control

CMD_CommandReq		0x0002

| No. | Field | TYPE | Length | description |
| --- | --- | --- | --- | --- |
| 1 | nGisIp | int | 4 | GIS User IP |
| 2 | nPort | short | 2 | GIS User port |
| 3 | sData | Char | 50 | Control command is ASCII TEXT： *KW Data start flag 000 ID； YYY command ID ； HHMMSS time， PARA1，PARA2paramter # end flag |

## Response of upload position

CMD_PosResp			0x8001

| No. | Field | TYPE | Length | description |
| --- | --- | --- | --- | --- |
| 1 | cErrorCode | char | 1 | 状态码 |

## Response of upload image

CMD_UploadFileResp			0x8200

| No. | Field | TYPE | Length | description |
| --- | --- | --- | --- | --- |
| 1 | cErrorCode | char | 1 | Status code |

## Response of upload image packet

CMD_SendJpgResp			0x8201

| No. | Field | TYPE | Length | description |
| --- | --- | --- | --- | --- |
| 1 | cErrorCode | char | 1 | Status code |

# Appendix

GPRS_Server will save the last IP and port of tracker, if the GIS client will send the control or set command ,it will send to gprs_server first, then server send to tracker, Tracker will send the position and alarm data to server ,server will save the data into the DB on the server ,if it is alarm message ,it will send to gis client.

## Get  tracker position * KW,000 ,000,HHMMSS#

Server send to tracker

| note:\r\n length is 2,HEX: 0x0d0a \r\n*KW (sMark:6) | 68 (nPackLen:2) | 0x0002 (CMD:2) | 234567 (nGisIp:4) | 1234 (nPort:2) |
| --- | --- | --- | --- | --- |
| *KW,000,000,070508# (sData:50) | “\r\n” (sEnd:2) |  |  |  |

respond: Tracker send to server

| 59 (packetLen:2) | 0x8009 (CMD:2) | 234567 (nGisIp:4) | 1234 (nPort:2) | 1 (bEnable:1) | 1 (bAlarm:1) |
| --- | --- | --- | --- | --- | --- |
| 20 (nSpeed:4) | 12 (nDirection:4) | 113.99907 (fLongitude:4) | 22.5477 (fLatitude:4) | NR09B00001 (sUserID:12) | 02-02-10 11:06:30 (sDateTime:18) |
| 1 (cErrorCode:1) |  |  |  |  |  |

## Modify  device password：* KW, 000,001, HHMMSS, ******,$$$$$$ #

******is old password（default is 000000）,$$$$$$ is new password

Example:

| \r\n*KW (sMark:6) | 68 (nPackLen:2) | 0x0002 (CMD:2) | 234567 (nGisIp:4) | 1234 (nPort:2) |
| --- | --- | --- | --- | --- |
| * KW,000,001,070508，000000,111111# (sData:50) | “\r\n” (sEnd:2) |  |  |  |

Respond:

| 59 (packetLen:2) | 0x8009 (CMD:2) | 234567 (nGisIp:4) | 1234 (nPort:2) | 1 (bEnable:1) | 1 (bAlarm:1) |
| --- | --- | --- | --- | --- | --- |
| 20 (nSpeed:4) | 12 (nDirection:4) | 113.99907 (fLongitude:4) | 22.5477 (fLatitude:4) | NR09B00001 (sUserID:12) | 02-02-10 11:06:30 (sDateTime:18) |
| 1 (cErrorCode:1) |  |  |  |  |  |

## Real-time respond  *KW, 000,002, HHMMSS, interval #

interval =0 is stop ，interval=[30 ~ 65535 second]

example:  60 second  send a position to server

| \r\n*KW (sMark:6) | 68 (nPackLen:2) | 0x0002 (CMD:2) | 234567 (nGisIp:4) | 1234 (nPort:2) |
| --- | --- | --- | --- | --- |
| *KW,000,002,070508,60# (sData:50) | “\r\n” (sEnd:2) |  |  |  |

Respond:

| 59 (packetLen:2) | 0x8009 (CMD:2) | 234567 (nGisIp:4) | 1234 (nPort:2) | 1 (bEnable:1) | 1 (bAlarm:1) |
| --- | --- | --- | --- | --- | --- |
| 20 (nSpeed:4) | 12 (nDirection:4) | 113.99907 (fLongitude:4) | 22.5477 (fLatitude:4) | NR09B00001 (sUserID:12) | 02-02-10 11:06:30 (sDateTime:18) |
| 1 (cErrorCode:1) |  |  |  |  |  |

## Telphone set: * KW, 000,003, HHMMSS,P,TelNumber#

P=[1,3]

TelNumber is the telephone number，max length is 16

| \r\n*KW (sMark:6) | 68 (nPackLen:2) | 0x0002 (CMD:2) | 234567 (nGisIp:4) | 1234 (nPort:2) |
| --- | --- | --- | --- | --- |
| *KW,000,003,070508,2,3,13500000001# (sData:50) | “\r\n” (sEnd:2) |  |  |  |

Respond:

| 59 (packetLen:2) | 0x8009 (CMD:2) | 234567 (nGisIp:4) | 1234 (nPort:2) | 1 (bEnable:1) | 1 (bAlarm:1) |
| --- | --- | --- | --- | --- | --- |
| 20 (nSpeed:4) | 12 (nDirection:4) | 113.99907 (fLongitude:4) | 22.5477 (fLatitude:4) | NR09B00001 (sUserID:12) | 02-02-10 11:06:30 (sDateTime:18) |
| 1 (cErrorCode:1) |  |  |  |  |  |

## Control  the  engine ：* KW, 000,007， HHMMSS,0#（0：close，1：open）

Example

| \r\n*KW (sMark:6) | 68 (nPackLen:2) | 0x0002 (CMD:2) | 234567 (nGisIp:4) | 1234 (nPort:2) |
| --- | --- | --- | --- | --- |
| * KW,000,007,070508，0 # (sData:50) | “\r\n” (sEnd:2) |  |  |  |

respond

| 59 (packetLen:2) | 0x8009 (CMD:2) | 234567 (nGisIp:4) | 1234 (nPort:2) | 1 (bEnable:1) | 1 (bAlarm:1) |
| --- | --- | --- | --- | --- | --- |
| 20 (nSpeed:4) | 12 (nDirection:4) | 113.99907 (fLongitude:4) | 22.5477 (fLatitude:4) | NR09B00001 (sUserID:12) | 02-02-10 11:06:30 (sDateTime:18) |
| 1 (cErrorCode:1) |  |  |  |  |  |

## Over speed set ：* KW, 000,005, HHMMSS ,XX#

XX is the speed

=00is close the over speed alarm function

=[01<XX<20]  (unit: 10Km)

Example: set the max speed is 10Km/H

| \r\n*KW (sMark:6) | 68 (nPackLen:2) | 0x0002 (CMD:2) | 234567 (nGisIp:4) | 1234 (nPort:2) |
| --- | --- | --- | --- | --- |
| *KW,000,005,070508,10 # (sData:50) | “\r\n” (sEnd:2) |  |  |  |

Respond:

| 59 (packetLen:2) | 0x8009 (CMD:2) | 234567 (nGisIp:4) | 1234 (nPort:2) | 1 (bEnable:1) | 1 (bAlarm:1) |
| --- | --- | --- | --- | --- | --- |
| 20 (nSpeed:4) | 12 (nDirection:4) | 113.99907 (fLongitude:4) | 22.5477 (fLatitude:4) | NR09B00001 (sUserID:12) | 02-02-10 11:06:30 (sDateTime:18) |
| 1 (cErrorCode:1) |  |  |  |  |  |

## e-Fence Setting: * KW, 000,006, HHMMSS ,XX#

XX = [00,50]  (unit :100m)

=00   is close e_Fence function

=01    100m

=02    200m

=03    300m

=04    400m

=05    500m

Example: Set Distance (1100)M

| \r\n*KW (sMark:6) | 68 (nPackLen:2) | 0x0002 (CMD:2) | 234567 (nGisIp:4) | 1234 (nPort:2) |
| --- | --- | --- | --- | --- |
| *KW,000,006,070508,11 # (sData:50) | “\r\n” (sEnd:2) |  |  |  |

Respond

note：(the command is Valid only when GPS work well

| 59 (packetLen:2) | 0x8009 (CMD:2) | 234567 (nGisIp:4) | 1234 (nPort:2) | 1 (bEnable:1) | 1 (bAlarm:1) |
| --- | --- | --- | --- | --- | --- |
| 20 (nSpeed:4) | 12 (nDirection:4) | 113.99907 (fLongitude:4) | 22.5477 (fLatitude:4) | NR09B00001 (sUserID:12) | 02-02-10 11:06:30 (sDateTime:18) |
| 1 (cErrorCode:1) |  |  |  |  |  |

## Set the device ID：* KW, 000,009, HHMMSS ,YYYY#

YYYY the device ID

例：

| \r\n*KW (sMark:6) | 68 (nPackLen:2) | 0x0002 (CMD:2) | 234567 (nGisIp:4) | 1234 (nPort:2) |
| --- | --- | --- | --- | --- |
| *KW,000,009,070508,001 # (sData:50) | “\r\n” (sEnd:2) |  |  |  |

Respond

| 59 (packetLen:2) | 0x8009 (CMD:2) | 234567 (nGisIp:4) | 1234 (nPort:2) | 1 (bEnable:1) | 1 (bAlarm:1) |
| --- | --- | --- | --- | --- | --- |
| 20 (nSpeed:4) | 12 (nDirection:4) | 113.99907 (fLongitude:4) | 22.5477 (fLatitude:4) | NR09B00001 (sUserID:12) | 02-02-10 11:06:30 (sDateTime:18) |
| 1 (cErrorCode:1) |  |  |  |  |  |

## Restart device：* KW, 000,099, HHMMSS #

例：

| \r\n*KW (sMark:6) | 68 (nPackLen:2) | 0x0002 (CMD:2) | 234567 (nGisIp:4) | 1234 (nPort:2) |
| --- | --- | --- | --- | --- |
| *KW,000,014,070508# (sData:50) | “\r\n” (sEnd:2) |  |  |  |

Respond

| 59 (packetLen:2) | 0x8009 (CMD:2) | 234567 (nGisIp:4) | 1234 (nPort:2) | 1 (bEnable:1) | 1 (bAlarm:1) |
| --- | --- | --- | --- | --- | --- |
| 20 (nSpeed:4) | 12 (nDirection:4) | 113.99907 (fLongitude:4) | 22.5477 (fLatitude:4) | NR09B00001 (sUserID:12) | 02-02-10 11:06:30 (sDateTime:18) |
| 1 (cErrorCode:1) |  |  |  |  |  |

## Shake hand message：

Example：

| 16 (packetLen:2) | 0x0000 (CMD:2) | NR09B00001 (sUserID:12) |
| --- | --- | --- |

Respond

| \r\n*KW (sMark:6) | 13 (nPackLen:2) | 0x8000 (CMD:2) | 1 (cErrorCode:1) | “\r\n” (sEnd:2) |
| --- | --- | --- | --- | --- |

Time explanation:

void GetTimestring(long nDateTime, char* sDateTime)

{

long ndate = 0;

ndate = nDateTime>>26;//Year

sDateTime[0] = ndate/10 +'0';

sDateTime[1] = ndate%10 +'0';

sDateTime[2] = '-';

ndate = nDateTime>>22&0x0f;//Month

sDateTime[3] = ndate/10 +'0';

sDateTime[4] = ndate%10 +'0';

sDateTime[5] = '-';

ndate = nDateTime>>17&0x1f;//Day

sDateTime[6] = ndate/10 +'0';

sDateTime[7] = ndate%10 +'0';

sDateTime[8] = ' ';

ndate = nDateTime>>12&0x1f;//Hour

sDateTime[9] = ndate/10 +'0';

sDateTime[10] = ndate%10 +'0';

sDateTime[11] = ':';

ndate = nDateTime>>6&0x3f;//Minute

sDateTime[12] = ndate/10 +'0';

sDateTime[13] = ndate%10 +'0';

sDateTime[14] = ':';

ndate = nDateTime&0x3f;//Second

sDateTime[15] = ndate/10 +'0';

sDateTime[16] = ndate%10 +'0';

sDateTime[17] = 0;

}