2929 Protocol and Data Transfer Format

Revision history

| Version number | Modify the content | Revised | Revision time |
| --- | --- | --- | --- |
| 2.0 | Add some data descriptions and add setting instruction sets | Chen XX | 2022-05-27 |

- Explanation of the instruction structure and name concept

The underlying data structure

| 2 | 1 | 2 | 4 | n | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- |
| packet head | Master signaling | Package length | Fake IP | content | verify | Packet tail |

Bag head, bag tail

Refers to the frame boundary of the data, with the packet header occupying a length of 2 bytes and the end of the packet occupying a length of 1 byte

The header is a fixed character (0x29 0x29), and the end of the packet is represented by (0x0D).

Package length

Refers to the length of the packet content, starting from the first byte after the packet length byte position until the end of the packet occupies 2 bytes

verify

Refers to the XORing of all bytes from the header of the packet until the previous byte is checked, accounting for 1 byte

Pseudo-IP address

Refers to a pseudo-IP address into which the device ID number is converted by a specified method. occupies 4 bytes

Illustrate:

Endpoints communicate only via UDP

- Server (platform) data delivery

- (0x21) Center confirmation package

[Function] Receives a valid UDP packet from the vehicle-mounted device, and the center issues an acknowledgment packet

[Going] terminal

[Communication mode] This packet is only delivered via UDP

[Data format].

| 29 | 29 | 21 | 00 | 5 |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| packet head |  | Master signaling | Package length |  | Received Packets Check value | Received Package Master Letter cause | Received Bun letter cause | verify | Packet tail |

[Parameter description] This instruction is also used as a UDP confirmation response package for the vehicle terminal, for most of the instructions sent by the terminal, the instruction must be answered, otherwise it will be retransmitted, resulting in increased GPRS traffic. Under normal circumstances, when a valid UDP packet is received from the terminal, the center should issue an acknowledgment packet

The subsignaling is the 10th byte of the received packet. If there is no subsignaling in the packet, the subsignaling byte bit is preserved.

[Example] The Centre receives: 29 29 84 00 0A 00 00 00 00 C4 E3 BA C3D0 0D

Center answer: 29 29 21 00 05 D0 84 C4 B4 0D

[Terminal Answer] No reply is required

[Terminal Return] None

- (0x3A) Issue dispatch text information

[Function] Dispatches dispatch text messages to designated on-board devices

[Going] On-board equipment

[Communication method] UDP

[Format]

| 29 | 29 | 3A | 00 | n |  |  |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| packet head |  | Master signaling | Package length |  | 伪IP |  |  |  | Scheduling information | verify | Packet tail |

[Parameter description] The information is issued in the form of ASCII code or GB2312 code, and the maximum content of the short message is 240 bytes

[Example]

The center sends a "HELLO" message

29 29 3A 00 0B B7 05 D0 1D 48 45 4C 4C 4F 0C 0D

[Terminal Answer] Terminal answer acknowledgment packet, 0x85 acknowledgement

[Terminal Return] None

Note: The platform can configure terminal related parameters through this command, and the configuration content is consistent with the SMS command

Refer to Appendix 4 of the platform to issue 3A command parameter data setting process

- (0xD7) Proofreading response

[Function] When the terminal applies for schooling, the platform answers the time to the car machine

[Going] terminal

[Communication method] UDP

[Format]

| 29 | 29 | D7 | 00 | 06 |  |  |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| packet head |  | Master signaling | Package length |  | 伪IP |  |  |  | Time | verify | Packet tail |

[Parameter description] The time is BCD code: year, month, day, hour, minute, second + reserved (0x00) a total of 7Byte

[举例] 29 29 D7 00 0D B7 05 D0 1D 19 03 16 11 05 22 00 9F 0D

[Terminal Answer] Vehicle-mounted terminal answer confirmation package

[Terminal Return] None

- The terminal sends instructions

1. (0x80) General location data

[Function] The terminal sends the current location information to the center of the platform

[Going] Platform Center

[Communication method] UDP

[Format]

| 29 | 29 | 80 | 00 | 28+N |  |  |  |  | 34Byte) | N byte |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| packet head |  | Master signaling | Package length |  | 伪IP |  |  |  | Location data | Extend the data | verify | Packet tail |

[Signaling Description]

34Byteis general location data, the data format refers to Appendix 1;

If the terminal sends this instruction, wait for the center to confirm the packet 0x21 reply, and resend it after timeout, and resend it three times

Extended data description: When an extended function is set through instruction delivery, the data segment is included when the instruction is returned, otherwise the segment is empty, and the terminal defaults to the empty segment

[Example]

29 29 80 01 3D DB 1A B0  39 22 05 27 16 17 31 02 24 10 04 11 41 64 46 00 00 00 00 FF 00 00 00 FF F7 52 FF FF FF FF FF FF 00 00 12 00 24 34 36 30 3B 30 30 3B 39 33 36 39 3B 34 39 34 34 32 00 05 00 04 29 22 00 00 04 00 08 05 D6 00 11 00 A3 59 47 30 32 44 5F 54 36 33 5F 56 30 2E 37 3B 00 06 00 A5 00 00 00 10 00 06 00 89 FF FF FF FF 00 24 00 A9 01 CC 00 04 24 99 13 4E 1F 24 99 13 12 1E 24 99 11 8C 12 24 99 0E 81 10 00 00 00 00 00 00 00 00 00 00 00 70 00 B9 05 32 63 3A 36 31 3A 30 34 3A 37 39 3A 64 33 3A 39 63 2C 2D 35 32 2C 66 30 3A 39 32 3A 62 34 3A 31 38 3A 63 63 3A 35 31 2C 2D 36 36 2C 63 63 3A 30 38 3A 66 62 3A 39 38 3A 62 37 3A 39 35 2C 2D 37 30 2C 33 30 3A 66 63 3A 36 38 3A 65 64 3A 62 32 3A 37 36 2C 2D 37 34 2C 39 63 3A 61 36 3A 31 35 3A 39 66 3A 64 30 3A 62 30 2C 2D 37 34 00 06 00 C5 00 00 10 08 00 16 00 FB 38 39 38 36 30 34 30 32 31 30 32 30 38 30 33 36 33 37 39 36 00 05 00 AE 03 00 1E 00 05 F0 00 01 00 1E 00 09 F0 01 22 05 27 16 46 35 00 08 0D

[Center answer] Center acknowledgement package

2. (0x85) Terminal confirmation package

[Function] The terminal answers the instructions issued by the platform center

[Going] Platform Center

[Communication mode] UDP sends the corresponding method to the terminal according to the communication type of the instruction issued

[Format]

| 29 | 29 | 85 | 00 | 28 |  |  |  |  | 34Byte) |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| packet head |  | Master signaling | Package length |  | 伪IP |  |  |  | Location data | verify | Packet tail |

[Signaling Description]:This command receives the acknowledgment packet as a platform (0x3A) command

[举例] 29 29 85 00 28 37 B7 37 B7 08 06 28 11 31 16 02 23 25 73 11 40 5 2 76 00 00 00 00 F8 00 00 C0 7F FC 7F 00 00 3C 00 00 00 00 34 73 0D

[Center Answer] Hub acknowledgement package

3. (0xD8) Set parameters for sending applications

[Function] Send an application to the platform center (0x3A), Note: The terminal will apply once each time it goes online

[Going] Platform Center

[Communication method] UDP

[Format]

| 29 | 29 | D8 | 00 | 06 |  |  |  |  |  | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| packet head |  | Master signaling | Package length |  | Fake IP |  |  |  | verify | Packet tail |

[Center Answer] 3A command (no need to issue 3A command when there are no parameters to set)

Note: The process refers to Appendix 3 of  the platform to issue 3A instruction parameter data setting process

Appendix 1: General Location Data Structures and Interpretations (34 byte)

YMDHMS WWWW JJJJ SS FF T LLL A B C D W W E R T Y U I

[08 06 05 14 26 34 02 23 25 56 11 40 52 81 01 20 01 54 F8 00 4E 20 7F FC 5F 00 00 3C 00 00 00 00 00 00]

YMDHMS: Year, month, day, hour, minute, second

Dates represent ranges: year, last two digits; month, 1-12; date, 1-31

Time represents the range: hours, 00-23; minutes, 00-59; seconds, 00-59

Year, month, day, hour, minute, and second are all encoded with compressed BCD.

[For example]: 6/5/08 14:26:34 is expressed as:

08H,06H,05H,104H,26H,34H

WWWW: Latitude 4 bytes

Latitude display range: 00 degrees 00.000 minutes - 89 degrees 59.999 minutes;

Compressed BCD encoding is used, but the highest bit is the sign bit. "Positive" means "north latitude", "negative" means "south latitude";

The unit of "cent" is: thousandths.

[For example]: 22 degrees 32.556 minutes north latitude is expressed as:

0x02232556

JJJJ: Longitude 4 bytes

Longitude indicates the range: 000 degrees 00.000 minutes - 179 degrees 59.999 minutes;

Compressed BCD encoding is used, but the highest bit is the sign bit. "Positive" means "East Meridian", "Negative" means "West Meridian";

The unit of "cent" is: thousandths.

[Example]: 114 degrees 05.281 minutes east longitude is expressed as: (Note: 1 degree = 60 minutes)

0x11405281

SS: Speed 2 bytes

Indicated range: 0-9999 km / h

Compressed BCD encoding is used.

[For example]: 120 km/h is expressed as:

0x0120

FF: Direction 2 bytes

Indication range: 000-359 degrees

Using compressed BCD coding, true north is 0 degrees, counting clockwise.

The unit is: degrees.

[For example]: 154 degrees is expressed as:

0x0154

T: positioning, antenna, power status : 1 byte

The positioning state consists of a single byte, which has the following meanings:

| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| D7 | Positioning flags |
| --- | --- |
| 0 | GPS is not located |
| 1 | GPS is located |
| D6 D5 |  |
| 1 1 | GPS is fine |
| 0 1 | The GPS antenna is open |
| D4 D3 | Power status |
| 1 1 | normal |
| 1 0 | The main power supply is powered down |
| 0 1 | The main supply voltage is too low |

| D2 | D1 | D0 | ACAR and A5 series terminals are not used |
| --- | --- | --- | --- |

LLL: (not used)

A B C D: Vehicle status with 4 bytes

A: (Terminal not used)

B: D7 D6 D5 D4 D3 D2 D1 D0

D7 D6 D5 D4 (Terminal not used)

D3 =0 Normal, D3 =1 Terminal removed

D2 = 0 reserved

D1=0 GPRS is online, D1=1 GPRS is not online

D0=0 terminal dialing succeeded, D0=1 terminal dialing was unsuccessful

C: D7 D6 D5 D4 D3 D2 D1 D0

D7=0 GPRS registered, D7=1 GPRS not registered

D6=0 The center does not need to issue 21 reply instructions, D6=1 The center should issue 21 reply instructions, indicating whether the center gives 21 reply instructions to this data, TCP mode does not need to answer

D5=0 UDP communication mode and D5=1 TCP communication mode represent the communication mode of the number of articles

D4 D3 D2 D1 D0 :CSQ 信号状态0-31

D: (not used)

W W E R T Y U (UNUSED)

I: 1 byte of the main command issued by the center

Appendix 2: Uplink (0x80) Extended Data Structures and Descriptions

- AD voltage value

| name | Length (2byte) | Directive (2byte) | Data (3byte) |
| --- | --- | --- | --- |
| AD voltage value | 0x0005 | 0x0004 | 29 22 00 |
| Example | Data: 00050004292200 Explanation: 29 22 00 means 2.92200V, generally reserved for two bits, that is, 2.92V |  |  |

- Battery remaining percentage value

| name | Length (2byte) | Directive (2byte) | Content (2byte) |
| --- | --- | --- | --- |
| The number of power bars | 0x0004 | 0x0008 | 05 D6 |
| Example | Data: 0004000805D6 Explanation: 05 D6 → 05D6 to decimal = 1494 The total number of devices is 1500, (1494/1500)%=99.60%. |  |  |

- Software version information

| name | Length (2 bYTE). | Directive (2byte) | Content (N byte). |
| --- | --- | --- | --- |
| Version number | N+2 | 00A3 | N |
| Example | Data: 001100A359473032445F5436335F56302E373B |  |  |

- Terminal model

| name | Length (2 bYTE). | Directive (2byte) | Content (4 bYTE). |
| --- | --- | --- | --- |
| Terminal model | 0x0006 | 0x00A5 | 00 00 00 10 |
| Example | Data: 000600A500000010 |  |  |

5.Extended alarm status bits

| name | Length (2 bYTE). | Directive (2byte) | Content (4 bYTE). |
| --- | --- | --- | --- |
| Alarm bits | 0x0006 | 0x0089 | FF FF FF FF |
| Example | Note: bit 12=1 normal, bit12=0 removed |  |  |

6. GSM base station (used in 2G terminals).

| name | Length (2 bYTE). | Directive (2byte) | Content (34 bytes) |
| --- | --- | --- | --- |
| Multiple base stations | 0x0024 | 0x00A9 |  |
| illustrate | V = country number, operator number, number of base stations, [ area code 1, tower number 1, signal strength 1,............ Area Code 6, Tower Number 6, Signal Strength 6] Country Code: Occupies 2 bytes, HEX denotes, e.g. 0x01CC denotes 460 Carrier number: occupies 1 byte, HEX means as 0x00 Number of base stations: occupies 1 byte, HEX means, such as 0x06, maximum support 6 base stations Area code: occupies 2 bytes, HEX means, high bit first, low bit last, such as 0x262C Tower number: occupies 2 bytes, HEX means that the high bit is first, the low bit is behind, as in 0x04BA Signal strength: occupies 1 byte, HEX denotes, such as 0x58, raw value Note: If there is no base station information, fill in 0 in the corresponding area code, tower number, and signal strength |  |  |

7. WiFi data

| name | Length (2 bYTE). | Directive (2byte) | content |
| --- | --- | --- | --- |
| Wifi hotspot | N+2 | 0x00B9 | It consists of the number of WIFI hotspots (1B) + WIFI hotspots (nB). |
| illustrate | Number of hotspots: 0x01 ~ 0x05, up to 5 groups, HEX denotes WIFI hotspot format: WIFI MAC address + signal value, ASCALL code representation For example: 24:69:68:5d:2c:a5,-30 Multi-group format: 24:69:68:5d:2c:a5,-30,2e:d0:5a:42:16:ad,-69,5c:0e:8b:8b:ca:50,-70,5c:0e:8b:8b:ca:52 ,-70,78:a1:06:6f:bb:fe,-70 Data sample: 00 70 00 B9 05 32 63 3A 36 31 3A 30 34 3A 37 39 3A 64 33 3A 39 63 2C 2D 35 32 2C 66 30 3A 39 32 3A 62 34 3A 31 38 3A 63 63 3A 35 31 2C 2D 36 36 2C 63 63 3A 30 38 3A 66 62 3A 39 38 3A 62 37 3A 39 35 2C 2D 37 30 2C 33 30 3A 66 63 3A 36 38 3A 65 64 3A 62 32 3A 37 36 2C 2D 37 34 2C 39 63 3A 61 36 3A 31 35 3A 39 66 3A 64 30 3A 62 30 2C 2D 37 34 |  |  |

8. Extended positioning status

| name | Length (2 bYTE). | Directive (2byte) | Content (4 byte). |
| --- | --- | --- | --- |
| Positioning status | 0x0006 | 0x00C5 | 00 00 10 08 |
| illustrate | State[31~0] Bit3/Bit4 positioning status: Bit3=0 Bit4=0 is not positioned Bit3=1 Bit4=0 GPS position Bit3=0 Bit4=1 WIFI position Note: When the Wifi is located, the terminal will report the MAC address to the server and obtain the latitude and longitude, so it can parse the latitude and longitude in the 0x80 location packet |  |  |

9.yes ICCID

| name | Length (2 bYTE). | Directive (2byte) | content |
| --- | --- | --- | --- |
| ICCID | 0x0016 | 0x00FB | 20 byte |
| Example | Data: 001600FB38393836303430323130323038303363373936 Character conversion 3839383630343032313032303830333633373936 89860402102080363796 |  |  |

10. Current working mode

| name | Length (2 bYTE). | Directive (2byte) | content |
| --- | --- | --- | --- |
| Working mode | N+2 | 0x00AE | N |
| illustrate | M+P M occupies 1 byte, M = 0x01 alarm mode, M = 0x03 tracking mode, M = 0x04 week mode When M=0x01, P represents the alarm time HHMM... HHMM, up to 4 groups, 2 bytes each For example, 0x1230 0x1330 means 12:30 and 13:30 respectively When M=0x04, P means that the week mode parameter = week mode switch (1Byte) + number of weeks (1Byte) + weeks (The number of bytes is determined by the number of weeks) + wake-up time point (2Byte). 040107010203040506071600 04 means week mode, 01 means week mode is on, 07 means seven days, and 01020304050607 means Monday Wake up until Sunday, 1600 means wake up at 16:00 Beijing time When M=0x03, P represents the return time TTTT (minutes), which occupies 2 bytes For example: 0x0063 means 99 minutes When M = 0x06, P means month mode parameter = month mode switch (1B) + number of timing (1B) + date (number of bytes Determined by the number of timings) + wake-up time (2B), where the number of ticks <=4, which means a maximum of 4 days per month For example: 060104010612261200 06 indicates the month mode, 01 means to enable the month mode, 04 means 4 days, and 01061226 indicates the 1st, 6th, 12th, and 26th of each month Wake up at 12:00 |  |  |

11. Preset reporting mode

| name | Length (2 bYTE). | Directive (2byte) | content |
| --- | --- | --- | --- |
| Reporting mode | N+2 | 0Xf000 | N |
| illustrate | Data format: Data format: Alarm Mode: mode(0x00)(1B)+t1(2B)+t2(2B)+t3(2B)+t4(2B) Timed callback: mode(0x01)(1B)+ts(2B) Week pattern: mode(0x02)(1B)+w(1B)+t(2B) Illustrate: Alarm Mode: mode is 00, the time is represented by the first byte for hours, the second byte for minutes, and BCD is encoded Example: 8:00 12:00 12:30 18:45 0x00 0x08 0x00 0x12 0x00 0x12 0x30 0x18 0x45 8:30 12:00 0x00 0x08 0x30 0x12 0x00 Timed callback: The mode is 01, and the interval value is expressed in minutes Example: Send back one in 3 minutes 0x01 0x00 0x03 999 minutes to return one message 0x01 0x03 0xE7 Week pattern: The mode is 02, and the week is set by bitwise 1 for setting and 0 for not setting. Position 0 means week 1, number 1 means week 2... The 6th place represents Sunday. The time is the same as the alarm clock mode Example: 1234 12:00 0x02 0x0F 0x12 0x00 1357 20:45 0x02 0x55 0x20 0x45 |  |  |

12. Next reporting time

| name | Length (2 bYTE). | Directive (2byte) | content |
| --- | --- | --- | --- |
| Next escalation time | 0x0009 | 0xF0001 | The time of the reported data (6 bytes) + the environment (1 byte). |
| illustrate | Format: First 6 bytes: YYMMDDHHmmss: year, month, day, hour, minute, second (BCD encoded). Last 1 byte: The environment refers to the GPS signal situation, showing all sky (0x00), half sky (0x01), basement (0x02) Illustrate: Date represents the range: Year, last two digits 00-99; Month, 1-12; Date, 1-31; Time represents the range: hours, 00-23; minutes, 00—59; seconds, 00—59; Example: 2018-11-01 12:45:36 0x18 0x11 0x01 0x12 0x45 0x36 The environment occupies 1 byte All sky (0x00). Half sky (0x01). Basement (0x02). |  |  |

13. Communication signal strength

| name | Length (2 bYTE). | Directive (2byte) | Content (1byte) |
| --- | --- | --- | --- |
| CSQ | 0x0003 | 0x0030 | 32-1, the higher the value, the better |

14. GNSS locates the number of stars

| name | Length (2 bYTE). | Directive (2byte) | Content (1byte) |
| --- | --- | --- | --- |
| GNSS locates the number of stars | 0x0003 | 0x0031 |  |

15. Single base station (4G LTE).

| name | Length (2 bYTE). | Directive (2byte) | Content (1byte) |
| --- | --- | --- | --- |
| Single base station | N+2 | 0x0024 | N |
| illustrate | Example: 00 16 00 24 34 36 30 3B 30 3B 39 33 36 39 3B 32 31 33 36 31 32 34 31 38 Escape: 460;0;9369;213612418 Four fields: Country, Operator, Area Code, Tower Code |  |  |

16. Single Base Station (2G GSM)

| name | Length (2 bYTE). | Directive (2byte) | Content (1byte) |
| --- | --- | --- | --- |
| Single base station | N+2 | 0x0024 | N |
| illustrate | Data includes four fields: country, operator code, area code, and tower number, to ; Separated by numbers: 460;01;270575;192749 Example: 29 29 80 00 40 0C 3A 3B 2A 00 00 00 08 00 00 02 23 20 31 11 40 15 70 00 00 00 00 38 00 00 00 FF FC 56 80 00 1E 00 00 00 00 00 00 00 16 00 24 34 36 30 3B 30 31 3B 32 37 30 35 37 35 3B 31 39 32 37 34 39 22 0D |  |  |

Appendix 3: Pseudo-IP and ID number Conversion Functions and Procedures

Note The  device ID number is an 11-digit number minus the first 1, for example: 13688888888 - 688888888

VB function

Public Function NumToIp(Num As Currency) As String

'Convert numbers into IP addresses

Dim Tmp As Currency

Dim Ip As String

Dim IpByte As Integer

Tmp = Int(Num / 256) *34447552

IpByte = Num - Tmp * 256

Ip = "." &CStr(IpByte)

Num = Tmp

Tmp = Int (Num / 256)

IpByte = Num - Tmp * 256

Ip = "." &CStr(IpByte) & Ip

Num = Tmp

Tmp = Int (Num / 256)

IpByte = Num - Tmp * 256

Ip = Tmp& "." &CStr(IpByte) + Ip

NumToIp = Ip

End Function

Public Function IpToNum(Ip As String) As Currency

'Convert IP addresses to numbers

Dim Num, iSum As Currency

Dim Tmp As String

Tmp= Ip

Num = CInt(mid(Tmp, 1, InStr(1, Tmp, ".") - 1))

iSum = Num * 256 * 256 * 256

Tmp = mid(Tmp, InStr(1, Tmp, ".") + 1)

Num = Val(mid(Tmp, 1, InStr(1, Tmp, ".") - 1))

iSum = iSum + Num * 256 * 256

Tmp = mid(Tmp, InStr(1, Tmp, ".") + 1)

Num = Val(mid(Tmp, 1, InStr(1, Tmp, ".") - 1))

iSum = iSum + Num * 256

Tmp = Val(mid(Tmp, InStr(1, Tmp, ".") + 1))

IpToNum = iSum + Tmp

End Function

Appendix 4: Platform Provisioning Process for 3A Command Parameter Data

Appendix 5: 0x80 Location Data Decomposition Sample and Platform Debugging Instructions

Platform analysis and positioning process:

a, the 28th byte top bit 7 represents the standard 2929 protocol location state, location data [16, 23].

b, Extended state 00C5 command indicates GPS or WIFI positioning, bit3/4=10: GPS positioning, bit3/4=01: WIFI positioning

bit3/4=Other values: Default GPS position

1, If the highest bit of the 28th byte is positioned, then determine what kind of positioning method the 00C5 extension is

2, if the highest bit of the 28th byte is 0 not positioned, in the 00C5 extended data, bit3/4=10 is determined to be GPS positioning, bit3/4=01 is judged to be WIFI positioning, and other values are judged to be not positioned, and the positioning data is [16,23].

3. If 1 and 2 items do not conform to the positioning, the WIFI data is judged first, and the platform server calls the third-party interface to resolve the WIFI location, and then resolves the base station data location if the resolution fails

The following location packets are determined to be WIFI positioning

292980013B14941494210417185424022409531141642500000000FF000000FFF750FFFFFFFFFFFFFFFF00001200243436303B30303B393336393B343934320005000429680000040008045F000F00A359475F434430315F52322E373B000600A50000001000060089FFFFFFFF002400A901CC00032499134E1C249913121C249911EF1C000000000000000000000000000000007000B90532633A36313A30343A37393A64333A39632C2D36372C63633A30383A66623A39383A62373A39352C2D37322C30383A39623A34623A39643A39623A35312C2D37332C39633A32313A36613A65343A31353A65382C2D37382C39633A61363A31353A39663A64303A62302C2D3833000600C500001010001600FB3839383630343132313031383730383434363635000500AE0300050005F0000100050009F00121041718590700D50D

Segmented parsing content:

2929 Baotou

80 Lord's New Order

013B packet length

14941494                   fake IP

210417185424022409531141642500000000FF000000FFF750FFFFFFFFFFFFFFFF00

Normal position data

001200243436303B30303B393336393B34393432 Primary base station

00050004296800 AD/voltage value

00040008045F Number of remaining power bars

000F00A359475F434430315F52322E373B0 Terminal debugging information

00600A500000010 Host model

00060089FFFFFFF extended alarm bit

002400A901CC00032499134E1C249913121C249911EF1C0000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000

0007000B90532633A36313A30343A37393A64333A39632C2D36372C63633A30383A66623A39383A62373A39352C2D37322C30383A39623A34623A39643A39623A35312C2D37332C39633A32313A36613A65343A31353A65382C2D37382C39633A61363A31353A39663A64303A62302C2D3833 wifi data

000600C500001010 Extended positioning status

001600FB3839383630343132313031383730383434363635 ICCID

000500AE030005 Current operating mode and wake-up time

0005F000010005 The escalation mode set up

0009F00121041718590700 Next wake-up time

D5 checksum

0D Packet Tail

Debugging protocol notes

# 1. Any time the platform receives the terminal .80 location package, be sure to give a reply

2. The platform determines whether the terminal is located depending on the positioning status uploaded back by the data package terminal, if the terminal transmits positioning, the platform does not do any processing, and directly uploads the latitude and longitude given by the device. (GPS positioning and wifi positioning, the data we give to the platform are processed, and the direct upload is latitude and longitude) If the platform wants to distinguish the positioning method, it will analyze 00C5 to distinguish whether the positioning method is satellite or wifi

3. The platform determines whether the terminal is located depending on the positioning status uploaded back by the data package terminal, if the terminal is not positioned, the platform needs to see whether the data brought up by the terminal has wifi MAC, if there is a platform to resolve the wifi address, the resolution is successful The positioning method will display wifi, if the resolution is not successful or there is no MAC in the packet, directly transmit the base station, the positioning method is base station positioning.