# GPS Protocol Data Definition

| Start of Data | IMEI_NUM | Command | Command/Data | End of Data | Check Sum |
| --- | --- | --- | --- | --- | --- |
| $1(2 bytes) | 15 bytes | 4 bytes | N Bytes | # | XOR of all bytes from $ to # |

# GPS Data from GPS Device to Server

# GPS Data

$1, IMEI_NUM,D001, VEHICLE_NUM, DD/MM/YY,HH:MM:SS,Latitude_DIR,Longitude_DIR,SPEED,COURSE,ODOMETER,IGNITION_STATUS,DIP1,DIP2,FUEL_ANA_VAL,GPS_STATUS,# CHECK_SUM

NOTE: The record fields separated with commas (,).

E.g. $1,357804047969310,D001,AP29AW0963,01/01/13,13:24:47,1723.9582N,07834.0945E,00100,010,0,0,0,0,0,A,#CS

# GPS Commands/Response between GPS Server and GPS Device

# GPS Device IP and Port Number Config Command and Response

$1, IMEI_NUM, C001, IP_NUM, PORT_NUM, #CS	(Command)

E.g. $1, 357804047969310,C001, 192.168.213.022, 5678, #CS

$1, IMEI_NUM, R001, STATUS,# CHECK_SUM	 (Response)

E.g. 	$1, IMEI_NUM, R001, 0, # CHECK_SUM     	-> Pass

$1, IMEI_NUM, R001, 1, # CHECK_SUM 		-> Fail

# GPS Device APN, UserID, Password Config Command

$1, IMEI_NUM, C002, APN, UID, PWD, # CHECK_SUM	(Command)

E.g. $1, 357804047969310, airtelgprs.com, xxx, xxx, # CHECK_SUM

$1, IMEI_NUM, R002, STATUS, # CHECK_SUM	 (Response)

E.g. 	$1, IMEI_NUM, R002, 0, # CHECK_SUM     	-> Pass

$1, IMEI_NUM, R002, 1,# CHECK_SUM 		-> Fail

# GPRS data interval frequency Command

$1, IMEI_NUM, C003, SECS, #CS	(Command)

E.g. $1, 357804047969310, 0060,#CS

$1, IMEI_NUM, R003, STATUS,# CHECK_SUM	 (Response)

E.g. 	$1, IMEI_NUM, R003, 0, # CHECK_SUM     	-> Pass

$1, IMEI_NUM, R003, 1, # CHECK_SUM 		-> Fail

# Configure GSM Phone numbers Command

$1, IMEI_NUM, C004, COUNTRY_CODE, SERVER_PHNO, VOICE_PHNO, SMS_PHNO, #CS	(Command)

E.g. $1, 357804047969310, C004, +91, 8790330567, 8790330567, 8790330567, #CS

$1, IMEI_NUM, R003, STATUS,# CHECK_SUM	 (Response)

E.g. 	$1, IMEI_NUM, R004, 0, # CHECK_SUM     	-> Pass

$1, IMEI_NUM, R004, 1, # CHECK_SUM 		-> Fail

# Configure Vehicle Number Command

$1, IMEI_NUM, C005,VEHICLE_NUM , #CS	(Command)

E.g. $1, 357804047969310,C005,AP29AW0963,#CS

$1, IMEI_NUM, R005, STATUS,# CHECK_SUM	 (Response)

E.g. 	$1, IMEI_NUM, R005, 0, # CHECK_SUM     	-> Pass

$1, IMEI_NUM, R005, 1, # CHECK_SUM 		-> Fail

# Get All Configuration Details Commands

$1, IMEI_NUM, C006, #CS	(Command)

E.g. $1, 357804047969310, C006, #CS

$1, IMEI_NUM, R005, STATUS, Vehicle Num., Server Num., Voice Num., SMS Num.,GPRS Freq. , <(TCP/UDP) Option for Sending>, <Primary Port Num.>, <Primary IP Num.>, <APN>, <User>, <Password>, # CHECK_SUM	 (Response)

E.g. 	$1, IMEI_NUM, R006, 0, Vehicle Num., Server Num., Voice Num., SMS Num.,GPRS Freq. , <(TCP/UDP) Option for Sending>, <Primary Port Num.>, <Primary IP Num.>, <APN>, <User>, <Password>, # CHECK_SUM     	-> Pass

$1, IMEI_NUM, R005, 0, # CHECK_SUM 		-> Fail