GPRS communication protocol between server and GPS tracker

Protocol:

| ! is start character | , is space character | ; is ending character |
| --- | --- | --- |

Command format of GPRS packets are as follows:

From tracker to server:

Form 1-1

| !1,gsn; | gsn:IMEI, the server will establish a connection according to IMEI. |
| --- | --- |
| !3,ok/error; | Set the parameters success/ failure |
| !4,f1,f2,f3,f4,f5,f6,f7,f8,f9; | Check status，Please refer to the form 1-3 state |
| !5,csq,sta; | Heartbeat,CSQ:0-31; sta:GPS signal,A means it has GPS signal,V means no GPS signal |
| !A,date,time,lat,lot,speed, Degrees,flag; | Real time data： !A,01/12/10,13:25:35,22.641724,114.023666,000.1,281.6,0; Date：01/12/10 //dd/mm/nn time：13:25:35 //hh:mm:ss lat ：22.641724 // lot ：114.023666 // speed：000.1 //speed degrees：281.6 //degree flag：14 // SOS Alarm state: see form1-2 |
| !B,data1#data2…#dataN; | N:1-15; dataN:date,time,lat,lot,speed,flag; History data |

Status：Form1-2 flag:(Decimal)

0： normal

1: input1

2: input2

3: input3

4: Over Speed Alarm

5: Geo Fence Alarm

6: External Power Lost

7: Low Power Alarm

8: input4

9: input5

10: ACC OFF

11: ACC On

12: Siren Alarm

13: Door Opened

14: SOS Alarm

From Server to tracker：

Form 1-3

| Operations: | Parameters: | Function Switch | State |
| --- | --- | --- | --- |
| Data transmission interval | 123456M,num | num:001-999s | f1 |
|  | 123456S0 | Close GPRS connection |  |
| Over speed Limit Alarm | 12346J1,num | num:001-250Km/h | f2 |
|  | 123456J0 | cancel |  |
| Set number A | 123456A1,num | num: (+86)+(num) | f3 |
|  | 123456A0 | cancel |  |
| Set number B | 123456B1,num | eg: 123456B1,+8613912345678 | f4 |
|  | 123456B0 | cancel |  |
| Set number C | 123456C1,num | num: (+86)+(tel) | f5 |
|  | 123456C0 | cancel |  |
| Modify the time zone | 123456Lnum | num: -11 to +11 | f6 |
| Engine control | 123456D | Stop engine | f7 |
|  | 123456E | cancel |  |
| Coordinates saving | 123456K1 | Function apply | f8 |
|  | 123456K0 | cancel |  |
| External power lost alarm | 123456N1 | Function apply | f9 |
|  | 123456N0 | cancel |  |
| Request status | 123465G | Request, the return parameters, Please refer to the “State” |  |

Firmware Upgrade Over Air Protocols:

| Read firmware version |  |  |  |
| --- | --- | --- | --- |
| send: |  |  |  |
| 123456V |  |  |  |
| receive: |  |  |  |
| !7,V1.06.1048,csq; //V1.06.1048:firmware verion , CSQ: GSM signal quality |  |  |  |

Send firmware: 518byte per frame

Server send:

Start 0x01 total-frames 0xxx frame order 0xxx 512 data data1 data2 ... data512 checksum 0xYY 0xyy end 0x04

Server receive:

!8,blockAll,blockNum,ok; //blockAll: totalframes blockNum: frame order(hex)

!9,blockAll,blockNum,error;

Note:

The software will check if the size of the chosen file is integer multiple of 518. If not, it will remind wrong file.

Every correct frame will get reply of OK; every incorrect frame will get reply of ERROR. All of them have corresponding total frames and frame order.

With reply of OK, it will send the next frame by the frame order. It will send all frames one by one. After the latest frame upgraded, it will remind that Upgrade success.

With reply of ERROR, it will send the same frame again. Also, if server gets no response in 20 seconds, it will send the same frame again. It will remind that Upgrade failed when the frame has been sent 3 times and failed.