Communication Protocol

VER:V1.7_C

\

Contents

1.The terminal commands	3

(1)Link remains	3

(2)Location data reporting	3

(3)Blind spot data transmission	3

(4)Alarming data reporting	4

(5)Address requsting instruction	4

(6)Request instruction of latitude and longtitude	4

2.Send command on platform	5

(1)Data uploading intervals settings	5

(2)Setting center number	5

(3)Assistance center number settings	5

(4)Control password settings	5

(5)Dial calls	6

(6)Send message	6

(7)Monitor	6

(8)SOS number settings	7

a. Set the first SOS number 	7

b.Set the second SOS number	7

c.Set the third SOS number	7

b.Set the forth SOS number	7

(9) Remote upgrading	8

(10)IP port settings	8

(11)Factory data reset	8

(12)Language and time area settings	8

(13)Query URL google link	9

(14)SOS message alarming switch	9

(15)Low-battery alarming switch	9

(16) APN settings	9

(17) SMS access control	10

(18) Parameter query	10

(19)Version quer	11

(20)Reboot	11

(21)Positioning order	11

(22)Bluetooth control order	11

(23)Working time directive	12

(24)Working time settings order	12

(25)Shutdown order	12

(26)Watch -taking -off alarming	12

(27)Pulse inquiry	13

3.Appendix	13

Appendix1 Location data instruction	13

All data in this protocol is in fixed format[Company*device ID*Content length*Content].The vendor identification is fixed to 2 byte,content length is ASSII code in 4 byte,and the high is in front with the low back,for example,FFFF represents the length of 65535.

## 1.The terminal commands

## 1.Link remains

(1)

The terminal sends:

[CS*YYYYYYYYYY*LEN*LK]

Example:[SG*8800000015*0002*LK]

Platform reply:

[CS*YYYYYYYYYY*LEN*LK]

Example:[SG*8800000015*0002*LK]

Instruction:The link remains will send in every 5 minutes.If the terminal does not receive any the reply data,then it will reconnect in every 5 minutes.

(2)

The terminal sending:

[CS*YYYYYYYYYY*LEN*LK,steps,rolling times,the percentage of battery amount]

Example:[SG*8800000015*000D*LK,50,100,100]

Platform reply:

[CS*YYYYYYYYYY*LEN*LK]

Example:[SG*8800000015*0002*LK]

Instruction :The link remains will send in every 5 minutes.If the terminal does not receive any the reply data,then it will reconnect in every 5 minutes.Both the above conditions are existed.

## 2.Location data reporting

The terminal sendig:

[CS*YYYYYYYYYY*LEN*UD,Location data(Appendix 1)]

Example:

[SG*8800000015*0087*UD,220414,134652,A,22.571707,N,113.8613968,E,0.1,0.0,100,7,60,90,1000,50,0000,4,1,460,0,9360,4082,131,9360,4092,148,9360,4091,143,9360,4153,141]

Platform reply:

No

InstructionThe terminal will report loction and statement information according to the settings without the reply of platform.

## 3.Blind spot data transmission

The terminal sending:

[CS*YYYYYYYYYY*LEN*UD2,Location data(Appendix 1)]

Example:

[SG*8800000015*0088*UD2,220414,134652,A,22.571707,N,113.8613968,E,0.1,0.0,100,7,60,90,1000,50,0000,4,1,460,0,9360,4082,131,9360,4092,148,9360,4091,143,9360,4153,141]

The platform reply:

No

Instruction :Transport the reporting data when there is no landing on platform.

## 4.Alarming data reporting

The terminal sending:

[CS*YYYYYYYYYY*LEN*AL,location data(Appendix 1)]

Example:

[SG*8800000015*0087*AL,220414,134652,A,22.571707,N,113.8613968,E,0.1,0.0,100,7,60,90,1000,50,0001,4,1,460,0,9360,4082,131,9360,4092,148,9360,4091,143,9360,4153,141]

Platform reply:

[CS*YYYYYYYYYY*LEN*AL]

Example:[SG*8800000015*0002*AL]

Instruction:The terminal will send alarming information to platform when there is emergency.If the terminal does not get the reply ,it will reports on time until getting the alarming confirmation.

## 5.Address requesting commands

The terminal sending:

[CS*YYYYYYYYYY*LEN*WAD,Language,Location data(Appendix 1)]

Example:

[SG*8800000015*008B*WAD,CH,220414,134652,A,22.571707,N,113.8613968,E,0.1,0.0,100,7,60,90,1000,50,0001,4,1,460,0,9360,4082,131,9360,4092,148,9360,4091,143,9360,4153,141]

The terminal reply:

[CS*YYYYYYYYYY*LEN*RAD,location type,address data]

Example:[SG*8800000015*000C*RAD,GPS,Corresponding address and information]

Instruction: The terminal address requesting commands,among which the CN represnts chinese,EN represents English,the address data is GB232 code,the location types are GPS positioning and BASE positioning.

## 6.Request instruction of latitude and longitude

The terminal sending:

[CS*YYYYYYYYYY*LEN*WG,location data(Appendix 1)]

Example:

[SG*8800000015*0087*WG,220414,134652,A,22.571707,N,113.8613968,E,0.1,0.0,100,7,60,90,1000,50,0001,4,1,460,0,9360,4082,131,9360,4092,148,9360,4091,143,9360,4153,141]

The terminal reply:

[CS*YYYYYYYYYY*LEN*RG,location type,latitude,latitude

identification ,longitude,longitude identification]

Example: [SG*8800000015*0021*RG,BASE,22.571707,N,113.8613968,E]

Instruction: Apply in the GPS unstated statement,and request latitude and longitude through the base station.

# 2.Send command on platform

## 1.Data uploading intervals settings

The terminal sending:

[CS*YYYYYYYYYY*LEN*UPLOAD,time intervals]

Example:[SG*8800000015*0009*UPLOAD,10]

The terminal reply:

[CS*YYYYYYYYYY*LEN*UPLOAD]

Example:[SG*8800000015*0006*UPLOAD]

Instruction:set the time intervals of the terminal reporting

## 2.Center number settings

The platform sending:

[CS*YYYYYYYYYY*LEN*CENTER,center number]

Example:[SG*8800000015*0012*CENTER,00000000000]

The terminal reply:

[CS*YYYYYYYYYY*LEN*CENTER]

Example:[SG*8800000015*0006*CENTER]

Instruction:Set the center number ,and send SMS commands by phone number.

## 3.Assistance number settings

The platform sending:

[CS*YYYYYYYYYY*LEN*SLAVE,assistance number]

Example:[SG*8800000015*0011*SLAVE,00000000000]

The terminal sending:

[CS*YYYYYYYYYY*LEN*SLAVE]

Example:[SG*8800000015*0005*SLAVE]

Instruction:Set the assistance center number ,and send SMS order by phone number.

## 4.Control password settings

The platform sending:

[CS*YYYYYYYYYY*LEN*PW,password]

Example:[SG*8800000015*0009*PW,111111]

The terminal reply:

[CS*YYYYYYYYYY*LEN*PW]

Example:[SG*8800000015*0002*PW]

Instruction:set the terminal password,and the non-center number needs to add password when sending order.

## 5.Dial calls

The platform sending:

[CS*YYYYYYYYYY*LEN*CALL,phone number]

Example:[SG*8800000015*0010*CALL,00000000000]

The terminal reply:

[CS*YYYYYYYYYY*LEN*CALL]

Example:[SG*8800000015*0004*CALL]

Instruction:Dial the corresponding phone number through this order.

## 6.Sending SMS

The platform sending:

[CS*YYYYYYYYYY*LEN*SMS,Message number,message contents]

Example:[SG*8800000015*001C*SMS,00000000000,123ABC How are you]

The terminal reply:

[CS*YYYYYYYYYY*LEN*SMS]

Example:[SG*8800000015*0003*SMS]

Instruction:Send SMS to corresponding phone number through this order,and the SMS content is sent in GB232 code.

## 7.Monitor

The platform sending:

[CS*YYYYYYYYYY*LEN*MONITOR]

Example:[SG*8800000015*0007*MONITOR]

The terminal reply:

[CS*YYYYYYYYYY*LEN*MONITOR]

Example:[SG*8800000015*0007*MONITOR]

Instruction :The terminal will dial the center number automatically.

## 8.SOS number settings

### (1)Set the first SOS number

The platform sending:

[CS*YYYYYYYYYY*LEN*SOS1,phone number]

Example:[SG*8800000015*0010*SOS1,00000000000]

The terminal reply:

[CS*YYYYYYYYYY*LEN*SOS1]

Example:[SG*8800000015*0004*SOS1]

### (2) Set the second SOS number

The platform sending:

[CS*YYYYYYYYYY*LEN*SOS2,phone number]

Example:[SG*8800000015*0010*SOS2,00000000000]

The terminal reply:

[CS*YYYYYYYYYY*LEN*SOS2]

Example:[SG*8800000015*0004*SOS2]

### (3) Set the third SOS number

The platform sending:

[CS*YYYYYYYYYY*LEN*SOS3,Phone number]

Example:[SG*8800000015*0010*SOS3,00000000000]

The terminal reply:

[CS*YYYYYYYYYY*LEN*SOS3]

Example:[SG*8800000015*0004*SOS3]

### (4)Set the forth SOS number

The platform sending:

[CS*YYYYYYYYYY*LEN*SOS,phone number,phone number,phone number]

Example :[SG*8800000015*0027*SOS,00000000000,00000000000,00000000000]

The terminal reply:

[CS*YYYYYYYYYY*LEN*SOS3]

Example:[SG*8800000015*0003*SOS]

Instruction:Set the SOS number,and send SMS and dial calls when there is emergency.

## 9.Remote upgrading

The platform sending:

[CS*YYYYYYYYYY*LEN*UPGRADE,URL address]

Example:[SG*8800000015*0039*UPGRADE,http://www.3g-elec.com/g29_updata/test/jt_ads.bin]

The terminal reply:

[CS*YYYYYYYYYY*LEN*UP]

Example:[SG*8800000015*0007*UPGRADE]

Instruction:Control the terminal remote upgrading.

## 10.IP Port settings

The platform sending:

[CS*YYYYYYYYYY*LEN*IP,IP or domain name,port]

Example:[SG*8800000015*0014*IP,113.81.229.9,5900]

The terminal reply:

The is no answer from the terminal to this order,disconnect the current connection directly and connect to new server.

Instruction:Set the IP and port connected to platform.

## 11.Factory data reset

The platform sending:

[CS*YYYYYYYYYY*LEN*FACTORY]

Example:[SG*8800000015*0007*FACTORY]

The terminal reply:

[CS*YYYYYYYYYY*LEN*FACTORY]

Example:[SG*8800000015*0007*FACTORY]

Instruction:The terminal will reset factory data.

## 12.Language and time area settings

The terminal sending:

[CS*YYYYYYYYYY*LEN*LZ,language ,Time Area]

Example:[SG*8800000015*0006*LZ,1,8]

The terminal reply:

[CS*YYYYYYYYYY*LEN*LZ]

Example:[SG*8800000015*0002*LZ]

Instruction:Set the language and time area.

## 13.Query the Google link

The platform sending:

[CS*YYYYYYYYYY*LEN*URL]

Example:[SG*5678901234*0003*URL]

The terminal reply:

[CS*YYYYYYYYYY*LEN*URL,Google link]

Example:[SG*5678901234*006B*URL,url:

http://maps.google.com.hk/maps?q=N22.571695,E113.861404

Locate date:2014-4-23

Locate time:18:16:59]

Instruction:Query the local URL address.

## 14.SOS message alarm switch

The platform sending:

[CS*YYYYYYYYYY*LEN*SOSSMS,0 or 1]

Example:[SG*5678901234*0008*SOSSMS,0]

The terminal reply:

[CS*YYYYYYYYYY*LEN*SOSSMS]

Example:[SG*5678901234*0006*SOSSMS]

Instruction: Set if sending SMS to SOS number when there is emergency(0:close,1:open)

## 15.Low-battery alarming swich

The platform sending:

[CS*YYYYYYYYYY*LEN*LOWBAT,0 or1]

Example:[SG*5678901234*0008*LOWBAT,1]

The terminal reply:

[CS*YYYYYYYYYY*LEN*LOWBAT]

Example:[SG*5678901234*0006*LOWBAT]

Instruction: Set if send SMS  to the senter number when the battery amount is low.(0:close,1:open)

## 16.APN settings

The platfoem sending:

[CS*YYYYYYYYYY*LEN*APN,APN name ,User name,code,User data]

Example:[SG*5678901234*0011*APN,cmnet,,,46000]

The terminal reply:

[CS*YYYYYYYYYY*LEN*APN]

Example:[SG*5678901234*0003*APN]

Instruction: Set the terminal APN parameter.

## 17.SMS access settings

The platform sending:

[CS*YYYYYYYYYY*LEN*ANY,0 or 1]

Example:[SG*5678901234*0005*ANY,0]

The terminal reply:

[CS*YYYYYYYYYY*LEN*ANY]

Example:[SG*5678901234*0003*ANY]

Instruction:set the SMS control access.

## 18 Parameter query

The platform sending:

[CS*YYYYYYYYYY*LEN*TS]

Eaxample:[SG*5678901234*0002*TS]

The terminal reply:

[CS*YYYYYYYYYY*LEN*TS,The version number of software;Device ID; IME number;IP;port;The center number;assistance center number;SOS1 number;SOS2 number;SOS3 number;reporting intervals; battery amount; language;time area;satellite number;GSM signal strength;LED Switch;code;]

Instruction : Query the terminal parameter.

Example: [SG*5678901234*00FC*TS,ver:G29_BASE_V1.00_2014.04.24_09.47.23;

ID:SG*5678901234;

imei:1234SG*56789012345;

url:113.81.229.9;

port:5900;

center:;

slave:;

sos1:;

sos2:;

sos3:;

upload:30S;

work mode:1;

bat level:3;

language:1;

zone:8.00;

GPS:NO(0);

GPRS:OK(89);

LED:OFF;

pw:123456;

]

## 19.Version Query

The platform sending:

[CS*YYYYYYYYYY*LEN*VERNO]

Example:[SG*8800000015*0005*VERNO]

The terminal sending:

[CS*YYYYYYYYYY*LEN*VERNO,version number]

Example:[SG*8800000015*0028*VERNO,G29_BASE_V1.00_2014.04.23_17.46.49]

Instruction:Query the terminal software version.

## 20.Reboot

The platform sending:

[CS*YYYYYYYYYY*LEN*RESET]

Example:[SG*5678901234*0005*RESET]

The terminal reply:

[CS*YYYYYYYYYY*LEN*RESET]

Example:[SG*5678901234*0005*RESET]

Instruction:The terminal reboot.

## 21.Location order

The terminal sending:

[CS*YYYYYYYYYY*LEN*CR]

Example:[SG*5678901234*0002*CR]

The terminal reply:

[CS*YYYYYYYYYY*LEN*RESET]

Example:[SG*5678901234*0002*CR]

Instruction:Awake the terminal GPS mode immediately,and it will be in location statement for a while.

## 22.Bluetooth control order

The terminal sending:

[CS*YYYYYYYYYY*LEN*BT,open or close(1,0)]

Example:[SG*5678901234*0004*BT,1]

The terminal reply:

[CS*YYYYYYYYYY*LEN*RESET]

Example:[SG*5678901234*0002*BT]

Instruction:Control the Bluetooth switch,1 is open ,0 is close.

## 23.The working time area directive

The platform sending:

[CS*YYYYYYYYYY*LEN*WORK,working time area]

Example:[SG*5678901234*0019*WORK,6-9,11-13,13-15,17-19]

The terminal reply:

[CS*YYYYYYYYYY*LEN*RESET]

Example:[SG*5678901234*0004*WORK]

Instruction :Set the working time area,and separate every area by comma.

## 24.The working time settings order

The paltform sending:

[CS*YYYYYYYYYY*LEN*WORKTIME,working time]

Example:[SG*5678901234*000A*WORKTIME,3]

The terminal reply:

[CS*YYYYYYYYYY*LEN*RESET]

Example:[SG*5678901234*0008*WORKTIME]

Instruction:Set the terminal working time ,and the unit is minutes.

## 25.Shutdown order

The platform sending:

[CS*YYYYYYYYYY*LEN*POWEROFF]

Example:[SG*5678901234*0008*POWEROFF]

The terminal reply:

[CS*YYYYYYYYYY*LEN*RESET]

Example:[SG*5678901234*0008* POWEROFF]

Instruction:Shutdown function.

## 26.Watch -taking-off alarming

The platform sending:

[CS*YYYYYYYYYY*LEN*REMOVE,0 or 1]

Example:[SG*5678901234*0008*REMOVE,1]

The terminal reply:

[CS*YYYYYYYYYY*LEN*REMOVE]

Example:[SG*5678901234*0006*REMOVE]

Instruction:Taking off alarming function.

## 27.Pulse query

The platform sending:

[CS*YYYYYYYYYY*LEN*PULSE]

Example:[SG*5678901234*0005*PULSE]

The terminal reply:

[CS*YYYYYYYYYY*LEN*PULSE,pulse beating number]

Example:[SG*5678901234*0008*PULSE,72]

Instruction:Pulse query

# 三.Appendix

## Appendix 1:Location data instruction

| Name | Example(ASII code) | Instruction |
| --- | --- | --- |
| Date | 120414 | (day month year)2014 year 4 month 12 day |
| Time | 101930 | (Time minute seconds)10 hour 19 minute 30 seconds |
| If position function or not | A | A:Position V:No position |
| Latitude | 22.564025 | Definite in DD.DDDDDD format,The latitude is :22.564025. |
| Latitude identification | N | N represent North latitude ,S represents South latitude. |
| Longitude | 113.242329 | Definite in DDD.DDDDDD format ,The longitude is :113.242329. |
| Longitude | E | E represents east longitude ,W represents west longitude |
| Speed | 5.21 | 5.21miles/hour. |
| Direction | 152 | The direction is in 152 degree. |
| Alititude | 100 | The unit is meter |
| Satellite | 9 | Represent the satellite number |
| GSM signal strength | 100 | Means the current GSM signal strength (0-100) |
| Battery | 90 | Means the current battery grade percentage. |
| Pedometer | 1000 | The steps is 1000 |
| Rolling times | 50 | Rolling number is 50 times. |
| The terminal statement | 00000000 | Represents in Hexadecimal string,and the meaning isas following:The high 16bit is alarming,the low 16 bit is statement. Bit(from 0) Meaning(1 effective) 0 Low-battery 1 Out of fence 2 In fence 3 Bracelet statement 16 SOS alarming 17 Low-battery alarming 18 Out of fence alarming 19 In fence alarming 20 Watch-take-off alarming |
| Base station number | 4 | Report the station number,0 is no reporting |
| Connect to station ta | 1 | GSM time delay |
| MCC country code | 460 | 460 represents china |
| MNC network number | 02 | 02 represents china mobile |
| The area code to connect base station | 10133 | Area code |
| Serial number to connect base station | 5173 | Base station serial code |
| The signal strength | 100 | Signal strength |
| The nearby station1 area code | 10133 | Area code |
| The nearby station 1 serial number | 5173 | Station serial number |
| The nearby base station 1 signal strength | 100 | Signal strength |
| The nearby station2 area code | 10133 | Area code |
| The nearby station 2 serial number | 5173 | Station serial number |
| The nearby base station 2 signal strength | 100 | Signal strength |
| The nearby station3 area code | 10133 | Area code |
| The nearby station 3 serial number | 5173 | Station serial number |
| The nearby base station 3 signal strength | 100 | Signal strength |