xexun API

Data Types

Big-endian.

A field value of 0xFFFFFFFF is treated as invalid.

Data Structure

Each packet (including ACK) consists of: Header + Total Length + Message Header + Message Body + CRC + Tail.

| Field | Bytes | Description |
| --- | --- | --- |
| Header | 1 | Header is fixed at 0xFC; a packet whose first byte is not 0xFC should be discarded. |
| Total Length | 2 | Message Header (11 bytes) + Message Body (n bytes); excludes the header, the length field itself, CRC, and tail. |
| Message Header | 11 | See table below |
| Message Body | n | Corresponds to the message type |
| CRC | 2 | CRC-16/CCITT-FALSE computed over Message Header + Message Body; the CRC field is stored big-endian. |
| Tail | 1 | Tail is fixed at 0xCF; a packet whose last byte is not 0xCF should be discarded. |

Total Length

| No. | Name | Type | Bytes | Parsing Method | Example: Raw | Example: Parsed |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Total Length | U16 | 2 | Hex to decimal | 00 0C | 12 |

Message Header

| No. | Name | Type | Bytes | Parsing Method | Example: Raw | Example: Parsed |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Protocol Version | U8 | 1 | Hex to decimal | 03 | 3 |
| 2 | Message Type | U8 | 1 | Use directly | 20 | 20 |
| 3 | Message Sequence No. | U8 | 1 | Hex to decimal | 20 | 32 |
| 4 | 0 + IMEI | BCD[8] | 8 | Drop the leading 0 to get the 15-digit IMEI | 08 62 77 10 75 42 67 52 | 862771075426752 |

3.4 CRC

| No. | Name | Type | Bytes | Parsing Method | Example: Raw | Example: Parsed |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | CRC Checksum | U16 | 2 | Hex to decimal | 1A 30 | 6704 |

Terminal Uplink Data (Message Type 0x20)

The message body may concatenate one or more sub-message bodies. Each sub-message body uses the following format:

| Field | Type | Bytes | Description |
| --- | --- | --- | --- |
| Message ID | U8 | 1 | / |
| Message Length | U8 | 1 | Number of bytes of Message Content |
| Message Content | * | * | Message Content |

GPS Body (0x64)

| No. | Name | Type | Bytes | Parsing Method | Example: Raw | Example: Parsed |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Message ID | U8 | 1 | Use directly | 64 | 64 |
| 2 | Message Length | U8 | 1 | Hex to decimal | 22 | 34 |
| 3 | Tracking timestamp | U32 | 4 | UTC seconds | 69 AF 74 4E | 2026-03-10 01:30:54 UTC |
| 4 | Latitude | DOUBLE | 8 | Hex to decimal degrees | 40 36 92 FF 46 0D 72 20 | 22.5742077 |
| 5 | Longitude | DOUBLE | 8 | Hex to decimal degrees | 40 5C 82 92 D1 DB 71 1C | 114.0402112 |
| 6 | Altitude (m) | FLOAT | 4 | Hex to decimal | 42 8C 99 9A | 70.3 |
| 7 | Ephemeris count | U8 | 1 | Hex to decimal | 45 | 69 |
| 8 | Satellite count | U8 | 1 | Hex to decimal | 1A | 26 |
| 9 | Average signal of the 4 strongest satellites | U8 | 1 | Hex to decimal | 2B | 43 |
| 10 | Speed (km/h, divide this value by 10) | U16 | 2 | Hex to decimal | 00 00 | 0 |
| 11 | Heading (degrees, divide this value by 10) | U16 | 2 | Hex to decimal | 00 00 | 0 |
| 12 | Accuracy factor (decimeters) | U16 | 2 | Hex to decimal | 00 0A | 10 |
| 13 | Tracking duration (seconds) | U8 | 1 | Hex to decimal | 03 | 3 |

WIFI Body (0x65)

| No. | Name | Type | Bytes | Parsing Method | Example: Raw | Example: Parsed |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Message ID | U8 | 1 | Use directly | 65 | 65 |
| 2 | Message Length | U8 | 1 | Hex to decimal | 2F | 47 |
| 3 | Sampling success timestamp | U32 | 4 | UTC seconds | 69 AF 81 B4 | 2026-03-10 02:44:56 UTC |
| 4 | WIFI count | U8 | 1 | Hex to decimal | 06 | 6 |
| 5 | WIFI MAC | U8[6] | 6 | Join as XX:XX:XX:XX:XX:XX | F4 BF BB 0D 4D F1 | F4:BF:BB:0D:4D:F1 |
| 6 | WIFI signal | I8 | 1 | Hex to decimal | D2 | -46 |
| 7 | WIFI MAC | U8[6] | 6 | Join as XX:XX:XX:XX:XX:XX | 1C B7 96 66 4A BF | 1C:B7:96:66:4A:BF |
| 8 | WIFI signal | I8 | 1 | Hex to decimal | D1 | -47 |
| 9 | WIFI MAC | U8[6] | 6 | Join as XX:XX:XX:XX:XX:XX | 2E 3A EB E9 AA D0 | 2E:3A:EB:E9:AA:D0 |
| 10 | WIFI signal | I8 | 1 | Hex to decimal | CF | -49 |
| 11 | WIFI MAC | U8[6] | 6 | Join as XX:XX:XX:XX:XX:XX | 20 3A EB E9 AA D0 | 20:3A:EB:E9:AA:D0 |
| 12 | WIFI signal | I8 | 1 | Hex to decimal | CE | -50 |
| 13 | WIFI MAC | U8[6] | 6 | Join as XX:XX:XX:XX:XX:XX | 48 9E BD 8B B0 C3 | 48:9E:BD:8B:B0:C3 |
| 14 | WIFI signal | I8 | 1 | Hex to decimal | CD | -51 |
| 15 | WIFI MAC | U8[6] | 6 | Join as XX:XX:XX:XX:XX:XX | 00 AD D5 DF 86 B5 | 00:AD:D5:DF:86:B5 |
| 16 | WIFI signal | I8 | 1 | Hex to decimal | CA | -54 |

LBS Cell Tower Body (0x66)

| No. | Name | Type | Bytes | Parsing Method | Example: Raw | Example: Parsed |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Message ID | U8 | 1 | Use directly | 66 | 66 |
| 2 | Message Length | U8 | 1 | Hex to decimal | 11 | 17 |
| 3 | Sampling timestamp | U32 | 4 | UTC seconds | 66 75 36 0C | 2024-06-04 15:13:00 UTC |
| 4 | MCC | U16 | 2 | Hex to decimal | 01 CC | 460 |
| 5 | MNC | U16 | 2 | Hex to decimal | 00 00 | 0 |
| 6 | LAC | U32 | 4 | Hex to decimal | 00 00 26 2E | 9774 |
| 7 | CID | U32 | 4 | Hex to decimal | 0A 82 AB 1A | 176335642 |
| 8 | RSSI | U8 | 1 | Hex to decimal | FF | Invalid |

LBS Position Body (0x67)

| No. | Name | Type | Bytes | Parsing Method | Example: Raw | Example: Parsed |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Message ID | U8 | 1 | Use directly | 67 | 67 |
| 2 | Message Length | U8 | 1 | Hex to decimal | 14 | 20 |
| 3 | Sampling timestamp | U32 | 4 | UTC seconds | 69 AF 81 B4 | 2026-03-10 02:44:56 UTC |
| 4 | Latitude | DOUBLE | 8 | Hex to decimal degrees | 40 36 92 2D 62 38 DA 3C | 22.571005 |
| 5 | Longitude | DOUBLE | 8 | Hex to decimal degrees | 40 5C 82 F3 A1 4C EC 42 | 114.046120 |

Beacon Body (0x68)

| No. | Name | Type | Bytes | Parsing Method | Example: Raw | Example: Parsed |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Message ID | U8 | 1 | Use directly | 68 | 68 |
| 2 | Message Length | U8 | 1 | Hex to decimal | 0E | 14 |
| 3 | Sampling timestamp | U32 | 4 | UTC seconds | 69 89 44 54 | 2026-02-09 02:20:04 UTC |
| 4 | WiFi/Bluetooth flag: 1: WiFi 2: Bluetooth 3: Beacon movement alarm | U8 | 1 | Hex to decimal | 02 | 2 |
| 5 | Beacon count | U8 | 1 | Hex to decimal | 01 | 1 |
| 6 | Beacon ID | U8[6] | 6 | Join as XX:XX:XX:XX:XX:XX | D5 21 3B 79 C0 4E | D5:21:3B:79:C0:4E |
| 7 | Beacon signal | I8 | 1 | Hex to decimal | CA | -54 |
| 8 | Beacon battery | U8 | 1 | Hex to decimal | 58 | 88% |
| 9+ | Multi-beacon extension | U8[6] + I8 + U8 | 8 bytes per group | Same as rules 6-8 | - | Up to 10 beacons retained |

Alarm Body (0x69)

| No. | Name | Type | Bytes | Parsing Method | Example: Raw | Example: Parsed |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Message ID | U8 | 1 | Use directly | 69 | 69 |
| 2 | Message Length | U8 | 1 | Hex to decimal | 06 | 6 |
| 3 | Alarm timestamp | U32 | 4 | UTC seconds | 69 AF 91 15 | 2026-03-10 03:33:41 UTC |
| 4 | Alarm ID | U8 | 1 | Hex to decimal | 01 | SOS alarm |
| 5 | Alarm data length | U8 | 1 | Hex to decimal | 00 | 0 |
| 6 | Alarm value | DATA | * | Parsed according to the alarm value type | - | - |

Alarm ID definitions:

- 1: SOS alarm, value empty

- 2: Power-bank low-battery alarm, value empty

- 3: Worn alarm, value empty

- 4: Not-worn alarm, value empty

- 5: Motion alarm, value is U16

- 6: Stationary alarm, value is U16

- 7: Fall alarm, value is U16

- 8: Impact alarm, value is U16

- 9: Beacon-detected alarm, value empty

- 10: Beacon-lost alarm, value empty

- 11: Vehicle power connected alarm, value empty

- 12: Vehicle power disconnected alarm, value empty

- 13: ACC on alarm, value empty

- 14: ACC off alarm, value empty

- 15: Car door open alarm, value empty

- 16: Car door close alarm, value empty

- 17: ICCID change, value is BCD[10]

- 18: GPS fault alarm, value empty

- 20: Temperature alarm, value 2 bytes: [0] status (1=overheat, 0=normal), [1] temperature value

Status Body (0x6A)

| No. | Name | Type | Bytes | Parsing Method | Example: Raw | Example: Parsed |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Message ID | U8 | 1 | Use directly | 6A | 6A |
| 2 | Message Length | U8 | 1 | Hex to decimal | 18 | 24 |
| 3 | Battery | U8 | 1 | Hex to decimal, unit % | 06 | 6% |
| 4 | Network connection duration | U16 | 2 | Hex to decimal | 00 06 | 6 |
| 5 | CSQ | U8 | 1 | Hex to decimal, range 0-31 | 1F | 31 |
| 6 | Tracking sequence no. | U8 | 1 | 0xFF for non-positioning data | 00 | 0 |
| 7 | Motion value | U8 | 1 | Motion value | 00 | 0 |
| 8 | Charging status | U8 | 1 | 1=charging, 0=not charging | 00 | 0 |
| 9 | Step count | U16 | 2 | Hex to decimal | 00 00 | 0 |
| 10 | Temperature | I8 | 1 | Hex to decimal | 15 | 21 |
| 11 | Respiration rate | U8 | 1 | Hex to decimal, 0xFF means invalid | FF | Invalid |
| 12 | Heart rate | U8 | 1 | Hex to decimal, 0xFF means invalid | FF | Invalid |
| 13 | Blood pressure - systolic | U8 | 1 | Hex to decimal, 0xFF means invalid | FF | Invalid |
| 14 | Blood pressure - diastolic | U8 | 1 | Hex to decimal, 0xFF means invalid | FF | Invalid |
| 15 | Blood oxygen | U8 | 1 | Hex to decimal, 0xFF means invalid | FF | Invalid |
| 16 | Fuel level | U8 | 1 | Hex to decimal, 0xFF means invalid | FF | Invalid |
| 17 | Status timestamp | U32 | 4 | UTC seconds | 69 AF 91 15 | 1773113621 |
| 18 | Device status bits | U8 | 1 | Binary bits | 01 | 00000001 |
| 19 | Device status bits (ignored) | U8 | 1 | Binary bits | 00 | 00000000 |
| 20 | Device status bits (ignored) | U8 | 1 | Binary bits | 00 | 00000000 |
| 21 | Device status bits (ignored) | U8 | 1 | Binary bits | 00 | 00000000 |

- Device status bits:

- bit0: Strap (1=detached, 0=attached)

- bit1: Motor (1=locked, 0=unlocked)

- bit2: Wire 1 status (1=connected, 0=disconnected)

- bit3: Wire 2 status (1=connected, 0=disconnected)

- bit4: Wire 3 status (1=connected, 0=disconnected)

- bit5: Motion status (1=moving, 0=stationary). The remaining two bits are reserved.

Heartbeat Body (0x6B)

| No. | Name | Type | Bytes | Parsing Method | Example: Raw | Example: Parsed |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Message ID | U8 | 1 | Use directly | 6B | 107 |
| 2 | Message Length | U8 | 1 | Hex to decimal | 01 | 1 |
| 3 | Heartbeat fixed value | U8 | 1 | Hex to decimal | 01 | 1 |

Version Body (0x6E)

| No. | Name | Type | Bytes | Parsing Method | Example: Raw | Example: Parsed |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Message ID | U8 | 1 | Use directly | 6E | 6E |
| 2 | Message Length | U8 | 1 | Hex to decimal | 90 | 144 |
| 3 | Flag bits (ignored) | U8 | 1 | Binary bits | 31 | 00110001 |
| 4 | Version | U8[32] | 32 | UTF-8 text, may be padded with 00 at the end | 41 41 41 32 36 30 33 31 30 30 35 7C 41 41 41 32 36 30 31 30 38 30 30 7C 58 32 31 00 00 00 00 00 | AAA26031005¦AAA26010800¦X21 |
| 5 | ICCID | BCD[10] | 10 | BCD parsing | 89 85 20 19 92 41 20 22 93 05 | 89852019924120229305 |
| 6 | Extended string length (ignored) | U8 | 1 | Hex to decimal | / | / |
| 7 | Extended string content (ignored) | STRING | * | / | / | / |

OBD Body (0x70)

| No. | Name | Type | Bytes | Parsing Method | Example: Raw | Example: Parsed |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Message ID | U8 | 1 | Use directly | 70 | 70 |
| 2 | Message Length | U8 | 1 | Hex to decimal | 21 | 33 |
| 3 | OBD communication protocol | U8 | 1 | Hex to decimal | 01 | ISO15765_4ST_500K |
| 4 | Fuel system status A | U8 | 1 | Hex to decimal | 00 | 0 |
| 5 | Fuel system status B | U8 | 1 | Hex to decimal | 00 | 0 |
| 6 | Fuel level % | U8 | 1 | Hex to decimal | 00 | 0% |
| 7 | Throttle position (0-100%) | U8 | 1 | Hex to decimal | 18 | 24% |
| 8 | Vehicle speed km/h | U8 | 1 | Hex to decimal | 9B | 155 |
| 9 | Engine speed (rpm) | U16 | 2 | Hex to decimal | 21 00 | 8448 |
| 10 | Engine run time (seconds) | U16 | 2 | Hex to decimal | 00 00 | 0 |
| 11 | Engine load % | U8 | 1 | Hex to decimal | 00 | 0 |
| 12 | Ambient temperature (Celsius) | I16 | 2 | Hex to decimal | FF D8 | -40 |
| 13 | Coolant temperature (Celsius) | I16 | 2 | Hex to decimal | 00 70 | 112 |
| 14 | Intake air temperature (Celsius) | I16 | 2 | Hex to decimal | FF D8 | -40 |
| 15 | Intake manifold pressure (kPa) | U8 | 1 | Hex to decimal | C4 | 196 |
| 16 | Air flow (g/s) | U16 | 2 | Hex to decimal | 01 D4 | 468 |
| 17 | Atmospheric pressure (kPa) | U8 | 1 | Hex to decimal | 00 | 0 |
| 18 | Confirmed fault code | U8[5] | 5 | ASCII converted to PXXXX | 50 30 33 32 30 | P0320 |
| 19 | Pending fault code | U8[5] | 5 | ASCII conversion; five 0xFF means none | FF FF FF FF FF | None |
| 20 | Run time since fault codes cleared (minutes) | U16 | 2 | Hex to decimal | 00 00 | 0 |

OBD communication protocol enumeration:

- 0: UNKNOW_PROTOCOL

- 1: ISO15765_4ST_500K

- 2: ISO15765_4ST_250K

- 3: ISO15765_4EXT_500K

- 4: ISO15765_4EXT_250K

- 5: ISO9141_2ADDR

- 6: ISO14230_4ADDR

- 7: ISO14230_4HL

Platform ACK After Receiving Terminal Uplink Data (Message Type 0x20)

The platform ACK message body is fixed as:

| No. | Name | Type | Bytes | Parsing Method | Example: Raw | Example: Parsed |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Body | U8 | 1 | Fixed 0x00 | 00 | 0 |
| 2 | Timestamp | U32 | 4 | UTC seconds, big-endian | 69 AF 74 50 | 1773106256 |

Note: The customer platform must reply correctly, otherwise the terminal will fail to switch platforms or keep retransmitting data.

The ACK message sequence number must match the message sequence number reported by the terminal.

Server Downlink Command (Message Type 0x21)

A server downlink command consists of: Header + Length + Message Header + Message Body (command text) + CRC + Tail.

Commands are case-insensitive. Multiple commands can be joined with #, and the terminal reply is separated by # in the same order.

Multiple commands must be executed serially in text order, and the replies must correspond one-to-one with the original command order.

On success the executed result text is returned. If a single command fails, only that result returns FAIL; an unsupported command returns unkown; other commands are unaffected.

Common Commands

| Function | Command Text | Description |
| --- | --- | --- |
| Set APN | APN=Internet | / |
| Tracking mode and frequency | tk=2,60,2,3600,0 | Param 1: 1=GPS first, 2=GPS only, 3=WIFI first; Param 2: tracking interval when moving, range 5-300 s; Param 3: report delay after tracking, range 0-240 s; Param 4: tracking interval when stationary, range 60-86400 s; Param 5: keep persistent connection, 1=persistent, 0=not persistent |
| Switch IP | ip=p.xexun.com:8899 | Domain p.xexun.com, port 8899 |
| Power off | of=1 | / |
| Reboot | rt=1 | / |
| Message | mg=123 | Devices with a screen can display the received message |
| Time zone | tz=8 | Range -12 to +12 |
| Set Bluetooth beacon tracking | blevlcfg={"bssid_list":["0c4b543f2901","7c1e0682d571"]} | Once set, the terminal prioritizes searching for this Bluetooth ID when tracking; upon finding it, it uploads to the platform and stops tracking. blevlcfg={} clears the Bluetooth ID. |
| Set WiFi hotspot tracking | wifivlcfg={"bssid_list":["0c4b543f2901","7c1e0682d571"]} | Once set, the terminal prioritizes searching for this WiFi ID when tracking; upon finding it, it uploads to the platform and stops tracking. wifivlcfg={} clears the WiFi ID. |
| Whitelist | Whitelist=13434787816 | Once set, the terminal can auto-answer calls; no country code is needed when setting. |
| Stationary alarm | Static time=600 | Alarm when stationary for 600 minutes; range 0-1024 |
| Impact alarm threshold | imp_alarm=500 | Impact alarm triggered when motion threshold reaches 500; range 0-65535 |
| GNSS mode | gpsmode=0011 | 0011=GPS+BD, 0001=GPS only, 0010=BD only |
| Tag network mode | nw_mode=0 / nw_mode=1 | 0=external network, 1=internal network |
| P02/P01 LED Command | led_blink=30 Blink for 30 seconds led_blink=0 Stop blinking |  |
| P02/P01 LED Bee Command | Beep 10 times bee=10 Stop beep bee=1 |  |
| P02/P01 LED Color Command | Red : chgcolor=255,0,0 Green:chgcolor= 0,255,0 White : chgcolor =255,255,255 other color can ask AI for this command |  |

X16-Specific Commands

| Function | Command Text | Description |
| --- | --- | --- |
| Power-bank binding | Related bank=[ID] | Bind the specified power-bank ID; ID 0 unbinds. |
| Digital key binding | Related key=[ID] | Bind the specified key ID; ID 0 unbinds. |
| Beacon binding | Related beacon=[ID] | Bind the specified beacon ID; ID 0 unbinds. |
| Key unlock | wear=0 | Remotely control the electronic lock to open |